3GPP TS 32.275 V18.0.0 (2024-04)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Charging management;

MultiMedia Telephony (MMTel) charging

(Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, UMTS, LTE, charging, management, MMTel

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	9

3	Definitions and abbreviations	12

3.1	Definitions	12

3.2	Symbols	12

3.3	Abbreviations	12

4	Architecture considerations	13

4.1	High level MMTel architecture	13

4.2	MMTel offline charging architecture	14

4.3	MMTel online charging architecture	15

4.4	MMTel converged charging architecture	16

5	MMTel charging principles and scenarios	17

5.0	Introduction	17

5.1	MMTel charging principles	17

5.1.0	Introduction	17

5.1.1	Supplementary services invocation	19

5.1.1.1	OIP charging	19

5.1.1.2	OIR charging	19

5.1.1.3	TIP charging	19

5.1.1.4	TIR charging	19

5.1.1.5	HOLD charging	19

5.1.1.6	CB charging	19

5.1.1.7	CDIV charging	19

5.1.1.8	CW charging	20

5.1.1.9	ECT charging	20

5.1.1.10	MWI charging	20

5.1.1.11	CONF charging	20

5.1.1.12	CCBS charging	21

5.1.1.13	CCNR charging	21

5.1.1.14	FA charging	22

5.1.1.15	MCID charging	22

5.1.1.16	CAT charging	22

5.1.1.17	CUG charging	22

5.1.1.18	PNM charging	23

5.1.1.19	CRS charging	23

5.1.1.20	Reverse charging	23

5.1.1.21	AoC charging	23

5.1.2	Supplementary services management by the user	23

5.1.3	OCS-provided announcements for MMTel service	23

5.1.4	CHF provided announcements for MMTel service	23

5.2	MMTel offline charging scenarios	24

5.2.2	Diameter message flows	24

5.2.2.0 	Introduction	24

5.2.2.1	Message flows - Successful cases and scenarios	24

5.2.2.1.0	Introduction	24

5.2.2.1.1	Originating Identification Presentation (OIP) charging	25

5.2.2.1.2	Originating Identification Restriction (OIR) charging	26

5.2.2.1.3	Terminating Identification Presentation (TIP) charging	27

5.2.2.1.4	Terminating Identification Restriction (TIR) charging	28

5.2.2.1.5	Communication Hold (HOLD) charging	29

5.2.2.1.6	Communication Barring – CB (ICB/ACB) charging	30

5.2.2.1.6.1	Communication Barring (CB) – ICB and Charging Data Request	30

5.2.2.1.6.2	Communication Barring (CB) – OCB	31

5.2.2.1.7	Communications Diversion (CDIV) charging	32

5.2.2.1.7.1	Communications Diversion (CDIV) – successful establishment	32

5.2.2.1.7.2	Communications Diversion (CDIV) – release	34

5.2.2.1.8	Communication Waiting (CW) charging	35

5.2.2.1.9	Explicit Communication transfer (ECT) charging	36

5.2.2.1.9.1	Explicit Communication transfer (ECT): Blind Transfer	36

5.2.2.1.9.2	Explicit Communication transfer (ECT): Release	40

5.2.2.1.10	Message Waiting Indication (MWI) charging	41

5.2.2.1.11	Conderence (CONF) Charging	42

5.2.2.1.11.0	Introduction	42

5.2.2.1.11.1	CONF charging – user creating a conference	42

5.2.2.1.11.2	CONF charging – user joining a conference	44

5.2.2.1.11.3	CONF charging – user inviting another user to a conference	45

5.2.2.1.11.4	CONF charging – user leaving a conference	46

5.2.2.1.11.5	Three-Party (3PTY) charging – successful establishment	47

5.2.2.1.11.6	Three-Party (3PTY) charging – release	50

5.2.2.1.12	CCBS charging	51

5.2.2.1.13	CCNR charging	54

5.2.2.1.14	Flexible Alerting (FA)	56

5.2.2.1.14.1	Flexible Alerting (FA) - establishment	56

5.2.2.1.14.2	Flexible Alerting (FA) – call release	57

5.2.2.1.15	Malicious Communication Identification (MCID)	58

5.2.2.1.16	Customized Alerting Tone (CAT)	60

5.2.2.1.17	Closed User Group (CUG)	62

5.2.2.1.17.1	Closed User Group (CUG): Originating	62

5.2.2.1.17.2	Closed User Group (CUG): Terminating	63

5.2.2.1.18	Personal Network Management (PNM)	65

5.2.2.1.19	Customized Ringing Signal (CRS)	67

5.2.2.1.20	Advice of Charge (AoC)	68

5.2.3	GTP' record transfer flows	68

5.2.4	BI CDR file transfer	68

5.3	MMTel online charging scenarios	69

5.3.1	Basic principles	69

5.3.2	Diameter message flows	69

5.3.2.0	Diameter message flows	69

5.3.2.1	Message flows - Successful cases and scenarios	70

5.3.2.1.0	Interaction with IMS-GWF	70

5.3.2.1.1	Communications Diversion (CDIV)	71

5.3.2.1.1.1	Communications Diversion (CDIV) – successful establishment	71

5.3.2.1.1.2	Communications Diversion (CDIV) – release	72

5.3.2.1.2	Flexible Alerting (FA)	73

5.3.2.1.3	Closed User Group (CUG)	75

5.3.2.1.3.1	Closed User Group (CUG): Originating	75

5.3.2.1.3.2	Closed User Group (CUG): Terminating	77

5.3.2.1.4	Conference (CONF)	79

5.3.2.1.4.0	Introduction	79

5.3.2.1.4.1	CONF – user creating a conference - ECUR mode	79

5.3.2.1.4.2	CONF – user creating a conference - SCUR mode	81

5.3.2.1.4.3	CONF – user joining a conference (SCUR mode)	82

5.3.2.1.4.6	CONF (3PTY) – successful establishment	85

5.3.2.1.5	Explicit Communication transfer (ECT)	87

5.3.2.1.5.0	Introduction	87

5.3.2.1.5.1	Explicit Communication transfer (ECT): Blind Transfer with sending SIP REFER	88

5.3.2.1.5.2	Explicit Communication transfer (ECT): Blind Transfer with 3PCC	91

5.3.2.1.5.3	Explicit Communication transfer (ECT): Release	93

5.3.2.2	Message flows - error cases and scenarios	93

5.4	MMTel converged charging scenarios	94

5.4.1	Basic principles	94

5.4.1.1	General	94

5.4.1.2	Applicable Triggers for MMTel	94

5.4.1.2.1	General	94

5.4.2	Message flows	96

5.4.2.1	Introduction	96

5.4.2.2	Void	97

5.4.2.3	Void	97

5.4.2.4	Void	97

5.4.2.5	Void	97

5.4.2.6	MMTEL supplementary services – PEC	97

5.4.2.7	MMTEL supplementary services – ECUR	97

5.4.2.8	MMTEL supplementary services – SCUR	97

5.4.3	CDR generation	98

5.4.3.1	Introduction	98

5.4.3.2	Triggers for CHF CDR	98

5.4.3.2.1	General	98

5.4.4	Ga record transfer flows	98

5.4.5	Bi CDR file transfer	98

6	Definition of charging information	99

6.1	Data description for MMTel offline charging	99

6.1.1	Rf message contents	99

6.1.1.0	Introduction	99

6.1.1.1	Charging Data Request message description	99

6.1.1.2	Charging Data Response message description	99

6.1.2	GTP' message contents	99

6.1.3	CDR description on the Bi interface	99

6.1.3.1	CDR field type	99

6.1.3.2	CDR triggers	100

6.1.3.3	MMTel-AS CDR content	100

6.2	Data description for MMTel online charging	100

6.2.1	Ro message contents	100

6.2.1.0	Introduction	100

6.2.1.1	Debit / Reserve Units Request message	100

6.2.1.2	Debit / Reserve Units Response message	101

6.3	MMTel charging specific parameters	101

6.3.1	Definition of MMTel charging information	101

6.3.1.0	Introduction	101

6.3.1.1	MMTel charging information assignment for Service Information	101

6.3.1.2	Definition of the MMTel Information	101

6.3.1.3	Support of MMTel Information in MMTel offline charging	102

6.3.1.4	Support of MMTel Information in MMTel online charging	104

6.4	Data description for MMTel converged charging	104

6.4.1	Message contents	104

6.4.1.1	General	104

6.4.1.2	Structure for the converged charging message formats	105

6.4.1.2.1	Charging Data Request message	105

6.4.1.2.2	Charging Data Response message	105

6.4.2	Definition of the MMTel converged charging information	105

6.4.2.1	General	105

6.4.2.2	Definition of MMTel charging information	106

6.4.2.3	Detailed message format for converged charging	106

6.4.2.4	Formal MMTel converged charging parameter description	108

6.4.2.4.1	MMTel charging CHF CDR parameters	108

6.4.2.4.2	MMTel charging resources attributes	108

Annex A (informative): Bibliography	109

Annex B (informative): Change history	110

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is part of a series of Technical Specifications (TSs) that specify charging functionality and charging management in 3GPP networks. The 3GPP core network charging architecture and principles are specified in TS 32.240 [1], which provides an umbrella for other charging management TSs that specify:

- 	the content of the CDRs per domain and subsystem (offline charging);

- 	the content of real-time charging messages per domain / subsystem (online charging);

- 	the functionality of online and offline charging for those domains and subsystems;

-	 the interfaces that are used in the charging framework to transfer the charging information (i.e. CDRs or charging events)

The complete document structure for these TSs is defined in TS 32.240 [1].

The present document specifies the offline, online and converged charging for MultiMedia Telephony (MMTel) service and supplementary services, based on the functional description of MMTel in TS 22.173 [200]. Charging for the following supplementary services is specified:

-	Communications Diversion (CDIV);

-	Communication Hold (HOLD);

-	CONFerence (CONF);

-	Message Waiting Indication (MWI);

-	Originating Identity Presentation (OIP) / Originating Identity Restriction (OIR);

-	Terminating Identity Presentation (TIP) / Terminating Identity Restriction (TIR);

-	Call Barring (CB);

-	Explicit Call Transfer (ECT);

-	Communication Wait (CW);

-	Completion of Communications to Busy Subscriber (CCBS);

-	Completion of Communications by No Reply (CCNR);

-	Malicious Communication Identification (MCID);

-	Customized Alerting Tone (CAT);

-	Closed User Group (CUG);

-	Personal Network Management (PNM);

-	Customized Ringing Signal (CRS);

-	Reverse Charging;

-	Advice of Charge (AoC).

Charging of these supplementary services is performed at the respective MMTel AS. The MMTel charging aspects are an extension of the basic IMS charging capabilities as specified in the TS 32.260 [20].

This charging description includes the offline, online and converged charging architecture and scenarios specific to the MMTel, as well as the mapping of the common 3GPP charging architecture specified in TS 32.240 [1] onto the MMTel. It further specifies the structure and content of the CDRs, and the charging events.

The present document is related to other 3GPP charging TSs as follows:

-	The common 3GPP charging architecture is specified in TS 32.240 [1];

-	The common IMS charging principles are specified in the TS 32.260 [20].

-	The parameters, abstract syntax and encoding rules for these CDR types are specified in TS 32.298 [51].

-	A transaction based mechanism for the transfer of CDRs within the network is specified in TS 32.295 [54].

-	The file based mechanism used to transfer the CDRs from the network to the operator's Billing Domain (e.g. the billing system or a mediation device) is specified in TS 32.297 [52].

-	The 3GPP Diameter application that is used for MMTel offline and online charging is specified in 
TS 32.299 [50].

-	The services, operations and procedures of charging, using Service Based Interface are specified in TS 32.290 [57].

-	The charging service of 5G system is specified in TS 32.291 [58].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; charging management; charging architecture and principles".

[2] - [19]	Void

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21] - [40]	Void.

[41]	3GPP TS 32.281: "Telecommunication management; Charging management; Announcement service".

[42] - [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] - [56]	Void.

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: " Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] - [68]	Void.

[69] 	3GPP TS 32.280: "Telecommunication management; Charging management; Advice of Charge (AoC) service".

[70] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.101: "Service aspects; Service principles".

[102]	3GPP TS 22.115: "Service aspects; Charging and billing".

[103]	3GPP TS 23.002: "Network architecture".

[104] - [199]	Void.

[200]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[201]	3GPP TS 24.173: "IMS Multimedia telephony communication service and supplementary services; Stage 3 ".

[202]	3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[203]	3GPP TS 24.605: "Conference (CONF) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[204]	3GPP TS 24.606: "Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[205]	3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[206]	3GPP TS 24.608: "Terminating Identification Presentation (TIP) and Terminating Identification Restriction (TIR)using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[207]	3GPP TS 24.610: "Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[208]	3GPP TS 24.611: "Anonymous Communication Rejection (ACR) and Communication Barring (CB)using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[209]	3GPP TS 24.615: "Communication Waiting (CW) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[210]	3GPP TS 24.616: "Malicious Communication Identification (MCID) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[211]	3GPP TS 24.623: "Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Simulation Services".

[212]	3GPP TS 24.629: "Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[213]	3GPP TS 24.642: "Completion of Communications to Busy Subscriber (CCBS) and Completion of Communications by No Reply (CCNR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[214]	3GPP TS 24.647: "Advice Of Charge (AOC) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[215]	3GPP TS 24.654: "Closed User Group (CUG) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[216]	3GPP TS 24.239: "Flexible Alerting (FA) using IP Multimedia (IM) Core Network (CN) subsystem, Protocol specification".

[217]	3GPP TS 24.182: "IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT); Protocol specification".

[218]	3GPP TS 23.259: "Personal Network Management (PNM); Procedures and information flows; Stage 2".

[219]	3GPP TS 24.183: "IP Multimedia Subsystem (IMS) Customized Ringing Signal CRS); Protocol specification".

[220]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[221]	3GPP TS 24.228: "Signalling flows for the IP multimedia call control based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[222] - [401]	Void.

[402]	IETF RFC 4006 (2005): "Diameter Credit-Control Application".

[403] - [499]	Void.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [100], TS 32.240 [1] apply.

## 3.2	Symbols

# For the purposes of the present document, the following symbols applies:

Bi	Reference point for the CDR file transfer from the IMS CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and CGF.

Rf	Offline Charging Reference Point between an IMS Network Entity or an AS and CDF.

Ro	Online Charging Reference Point between an AS or MRFC and IMS-GWF and the OCS.

Nchf	Service based interface exhibited by CHF.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations defined in TR 21.905 [100], TS 32.240 [1] and the following apply:

3PCC	Third Party Call Control

3PTY	Three-Party

AoC	Advice of Charge

CAT	Customized Alerting Tone

CB	Communication session Barring

CCBS	Completion of Communication sessions to Busy Subscriber

CCNR	Completion of Communications on No Reply

CD	Communication Deflection

CDIV	Communication DIVersion

CFB	Communication Forwarding Busy

CFU	Communication Forwarding Unconditional

CFNL	Communication Forwarding on Not Logged-in

CFNR	Communication Forwarding No Reply

CFNRc	Communication Forwarding on Subscriber Not Reachable

CFUDB	Communication Forwarding User Determined Busy

CONF	CONFerence

CUG	Closed User Group

CW	Communication Waiting

ECT	Explicit Communication Transfer

FA	Flexible Alerting

HOLD	Communication HOLD

ICB	Incoming Communication Barring

MCID	Malicious Communication Identification

MMTel	MultiMedia Telephony

MWI	Message Waiting Indication

NDUB	Network Determined User Busy

OCB	Outgoing Communication Barring

OIP	Originating Identity Presentation

OIR	Originating Identity Restriction

PNM	Personal Network Management

CRS	Customized Ringing Signal

TIP	Terminating Identity Presentation

TIR	Terminating Identity Restriction

URN	Uniform Resource Name

# 4	Architecture considerations

## 4.1	High level MMTel architecture

Figure 4.1.1 depicts the IMS reference architecture, as described in TS 23.002 [103], with "colorized" AS as entity involved in MMTel service and supplementary services charging described in the present document.

# Figure 4.1.1: Entities involved in MMTel service charging in IMS logical architecture

The AS provides the application level network functionality for MMTel service and supplementary services, whereas the overall IMS Network provides the basic IMS capabilities supporting MMTel service.

## 4.2	MMTel offline charging architecture

Figure 4.2.1 depicts the MMTel offline charging architecture.

# Figure 4.2.1: MMTel offline charging architecture

This MMtel offline charging architecture is based on the IMS offline charging architecture described in TS 32.260 [20], with service CTFs supporting MMtel specific service charging, interfacing the CDF through the Rf reference point.

The CTFs considered in the MMTel offline charging architecture reside in the Application level network functionality providing MMTel service and supplementary services.

The CTFs related to charging for the IMS basic capabilities supporting MMTel service, are described in TS 32.260 [20], and reside in the set of IMS Nodes (S-CSCF, MRFC…) reflected in IMS offline architecture.

## 4.3	MMTel online charging architecture

The architecture for MMTel online charging is described in figure 4.3.1..

# Figure 4.3.1: MMTel online charging architecture

This MMtel online charging architecture is based on the IMS online charging architecture described in TS 32.260 [20], with service CTFs supporting MMtel specific service charging, interfacing the OCS through the Ro reference point.

The CTFs considered in the MMTel online charging architecture reside in the Application level network functionality providing MMTel service and supplementary services.

The CTFs related to charging for the IMS basic capabilities supporting MMTel service, are described in TS 32.260 [20], and reside in the set of IMS Nodes (S-CSCF, MRFC…) reflected in IMS online architecture.

## 4.4	MMTel converged charging architecture

The architectural options for MMTel converged charging are depicted in figure 4.4.1

Figure 4.4.1: MMTel converged charging architecture

This MMTel converged charging architecture is based on the IMS converged charging architecture described in TS 32.260 [20], with service CTFs supporting MMTel specific service charging, using the Nchf service. This architecture is also applicable to offline only charging.

The CTFs considered in the MMTel converged charging architecture reside in the Application level network functionality providing MMTel service and supplementary services, other CTFs related to charging for the IMS basic capabilities (supporting MMTel service), are described in TS 32.260 [20].

The general architecture components can be found in TS 32.240 [1].

Ga is described in clause 5.2.3 and Bi in clause 5.2.4 of this document, and Nchf is described in TS 32.290 [57].

# 5	MMTel charging principles and scenarios

## 5.0	Introduction

There are a variety of multimedia telephony supplementary services implemented at different IMS nodes. All the services should support subscription based charging, and some also consumption based charging. The subscription based charging is out of the scope of the present document.

Table 5.0.1 summarizes which of the services are applicable for offline, online and converged charging.

# Table 5.0.1: Relevance of the services for offline, online and converged charging

| Supplementary services | Offline charging | Online charging | Converged charging |
| --- | --- | --- | --- |
| OIP | X | - | X |
| OIR | X | - | X |
| TIP | X | - | X |
| TIR | X | - | X |
| CW | X | - | X |
| HOLD | X | - | X |
| CB | X | - | X |
| MWI | X | - | X |
| CONF | X | X | X |
| CCBS | X | - | X |
| CCNR | X | - | X |
| CDIV | X | X | X |
| ECT | X | X | X |
| FA | X | X | X |
| MCID | X | - | X |
| CAT | X | - | X |
| CUG | X | X | X |
| PNM | X | - | X |
| CRS | X | - | X |
| AoC | X | - | - |

## 5.1	MMTel charging principles

### 5.1.0	Introduction

The MMTel charging encompasses the Multimedia telephony service (e.g. multimedia conversational communications between two or more users, with speech as a typical usage, and also others combinations of media) together with the associated supplementary services charging aspects.

Every Supplementary services described in TS 22.173 [200] are subject to be involved in MMTel charging function description.

MMTel service and supplementary services charging function focuses on charging information provided by  the CTFs supporting MMtel specific service charging: calling user identification, called user identification, media component characteristics  and usage (speech only, speech with other component, add/retrieve components..), supplementary services applied. It enables to apply different flexible charging based on supplementary service type and options.

All the CTFs supporting MMtel specific service charging (AS) pertain to IMS domain, and as explained in TS 32.260 [20],  it is possible to correlate session/transaction related charging data generated from these different Nodes (AS), and others IMS Nodes involved in the session used for MMTel service handling , based on IMS Charging Identifier (ICID).

For AS(s) providing MMTel specific service and supplementary services, the "OneChargingSession" option, as described in TS 32.260 [20] can be applied, when acting as a B2BUA with IMS Charging Identifier (ICID) preserved between dialogs.

### 5.1.1	Supplementary services invocation

#### 5.1.1.1	OIP charging

The Originating Identification Presentation (OIP) service provides the terminating user with the possibility of receiving trusted (i.e. network-provided) identity information in order to identify the originating user.

#### 5.1.1.2	OIR charging

The Originating Identification Restriction (OIR) service is a service offered to the originating user. It restricts presentation of the originating user's identity information to the terminating user.

#### 5.1.1.3	TIP charging

The Terminating Identification Presentation (TIP) service provides the originating party with the possibility of receiving trusted information in order to identify the terminating party.

#### 5.1.1.4	TIR charging

The Terminating Identification Restriction (TIR) is a service offered to the terminating party which enables the terminating party to prevent presentation of the terminating identity information to originating party.

#### 5.1.1.5	HOLD charging

The Communication Hold supplementary service enables a user to suspend the media stream(s) of an established IP multimedia session, and resume the media stream(s) at a later time.

The charged parties may be any of the Hold parties. These roles are:

-	Calling Party;

-	Called Party;

Editor's note: When one of the parties originates a new call to a third party and the new call overbooks the bearer resource reserved by the held call, overbooking may be taken into account in charging. The solution for that is FFS.

#### 5.1.1.6	CB charging

The Communication Barring (CB) service offers the following services:

-	The Incoming Communications Barring (ICB) is a service that rejects incoming communications that fulfil certain provisioned or configured conditions on behalf of the terminating user.

-	The Anonymous Communication Rejection (ACR) is a particular case of the ICB service, that allows barring of incoming communications from an anonymous originator on behalf of the terminating user.

-	The Outgoing Communication Barring (OCB) is a service that rejects outgoing communications that fulfil certain provisioned or configured conditions on behalf of the originating user.

#### 5.1.1.7	CDIV charging

The Communications Diversion (CDIV) services enables the diverting user, to divert the communications addressed to the diverting user to another destination.

There are three actors active in a CDIV service of one diversion with the following roles:

-	Diverting user; the party that initiates the diversion of an incoming communication;

-	Originating user; the party which has initiated the communication and that stays in the communication which is diverted;

-	Diverted to user;

In case there is another diversion, the user that was first the diverting user (user B), is the originating user in the second diversion. The diverted to user of the first diversion (user C) is the diverting user of the second diversion.

The charging of the diverting user for the B-C leg is measured by the MMTel AS handling the CDIV service, i.e. by the MMTel AS of the diverting user (user B). For the diverting user a separate charging dialog (online and/or offline charging) is needed. Charging of the originating and the terminating user (user C) is done in alignment with the TS 32.260 [20].

Typically the diverting user is charged for the forwarded leg (B-C leg), however, normal roaming charging principles apply for the diverted to user also in case of communication diversion. If there is another diversion, the user that performs the second diversion (user C) is charged (typically) for the new forwarded leg (C-D leg).

Charging at the S-CSCF is done in alignment with the TS 32.260 [20] but the S-CSCF has no knowledge of the CDIV service.

#### 5.1.1.8	CW charging

The Communication Waiting (CW) service enables the application server to indicate to the subscriber, that there is at least one new communication is requested, and that no resources are available for that incoming communication. The user has then the choice of accepting, rejecting or ignoring the incoming communication. The maximum number of communications that may be waiting is a service provider option. If the current number of communications waiting is equal to the maximum, then any new attempted incoming communication request shall be rejected with a busy cause.

#### 5.1.1.9	ECT charging

The Explicit Communication Transfer (ECT) service provides a party involved in a communication to transfer that communication to a third party.

There are three actors active in a transfer, they are acting in the following roles:

-	Transferor: the party that initiates the transfer of the active communication that it has with the Transferee;

-	Transferee: the party which stays in the communication which is transferred;

-	transfer target: the party which the communication is transferred to and which replaces the Transferor in the communication.

- The charging of the Transferor is measured by the MMTel AS handling the ECT service for the Transferor. 
The charging of the Transferee is measured by the MMTel AS handling the ECT service for the Transferee.

#### 5.1.1.10	MWI charging

The Message Waiting Indication (MWI) service enables the application server to indicate to the subscriber, that there is at least one message waiting. The indication is delivered to the subscriber's UE after successful subscription to the MWI service as described in the present document.

#### 5.1.1.11	CONF charging

The Conference (CONF) service enables a user to participate in and control a simultaneous communication involving a number of users.

CONF Charging for the conference owner could be based on:

-	establishment of the conference;

-	number of participants;

-	duration.

CONF Charging for the conference participants could be based on:

-	duration.

The charging of the conference owners and participants is measured by the SIP AS and MRFC handling conference service. For each participant (call leg) a separate charging dialog at the SIP AS is needed.

#### 5.1.1.12	CCBS charging

The Completion of Communication to Busy Subscriber (CCBS) service enables user A, encountering a busy destination B, to have the communication completed without the user having to manually initiate a new communication attempt when the destination B becomes not busy.

When user A requests the CCBS supplementary service, the network monitors for destination B becoming free again.

When destination B becomes free again, the network waits a short time in order to allow the resources to be re-used for originating a communication. If the resources are not re-used by destination B within this time, the network automatically recalls user A.

When user A accepts the CCBS recall, the network automatically generates a CCBS call to destination B.

#### 5.1.1.13	CCNR charging

The Completion of Communications on No Reply (CCNR) service enables user A, encountering a destination B which does not answer the communication (No Reply), to have the communication completed without the user having to manually initiate a new communication attempt when the destination becomes not busy after having initiated and completed a new communication.

When user A encounters a destination B which does not answer the communication (No Reply), user A can request the CCNR supplementary service.

When user A requests the CCNR supplementary service, the network monitors for destination B becoming not busy after having initiated and completed a new communication.

When destination B becomes not busy after having initiated and completed a new communication, the network waits a short time (as defined by the destination B idle guard timer) in order to allow the resources to be reused for originating a communication. If the resources are not reused by destination B within this time, the network automatically recalls user A.

When user A accepts the CCNR recall, then the network  automatically generates a CCNR call to destination B.

#### 5.1.1.14	FA charging

Flexible Alerting (FA) causes a call to a "Pilot Identity" to branch the call into several legs to alert several termination addresses (FA group members) simultaneously. 
Additional calls may be delivered to the FA Pilot Identity at any time. 
The first leg to be answered is connected to the calling party. The other call legs are abandoned.

The FA group, identified by the "Pilot Identity" consists of a list of FA group members alerted through their the public addressable identity.

#### 5.1.1.15	MCID charging

The Malicious Communication Identification (MCID) service enables an incoming communication to be identified as malicious and registered.

The Network shall register the communication related information (such as Terminating Identity Information, Originator Identity Information, Local Time and Date…), which shall be kept under Network Operator's control (i.e. not available to the terminating entity nor the originating entity).

This service has two modes: permanent mode and temporary mode. The permanent mode is active for all incoming communications, and the temporary mode is active only for the incoming communications declared by the served user.

The MCID service can be invoked during the active phase of the communication, or after the active phase for a limited period (but never after communication termination) by the served user, or, automatically invoked during the alerting phase.

#### 5.1.1.16	CAT charging

The Customized Alerting Tone Service (CAT) service is an operator specific service by which an operator enables the subscriber to customize the alerting tone which is played to the calling party.

#### 5.1.1.17	CUG charging

The Closed User Group (CUG) service enables users to form groups of members, whose communication profile is restricted for incoming and outgoing communications. Members of a specific CUG can communicate among themselves but not, in general, with users outside the group.

Specific CUG members can have additional capabilities that allow them to initiate outgoing communications to users outside the group, and/or to accept incoming communications from users outside the group. Specific CUG members can have additional restrictions that prevent outgoing communications to other members of the CUG, or prevent incoming communications from other members of the CUG.

A specific user may be a member of one or more CUGs.

#### 5.1.1.18	PNM charging

The Personal Network Management (PNM) service allows a user to manage his UEs in regard to terminating services according to preferences set by the user, capabilities and availabilities of devices. Charging for the "PN UE redirection" functionality of the defined PNM service, is considered in the scope of MMTel supplementary service charging, and applies to the active UE the terminating services are delivered to.

#### 5.1.1.19	CRS charging

The Customized Ringing Signal (CRS) service is an operator specific service by which an operator enables the subscriber to customize the ringing signal which is played to the called party.

#### 5.1.1.20	Reverse charging

The Reverse charging at communication set up time service allows the terminating party to be charged for the entire communication. The service, requested by the originating party at communication set up time, is explicitly indicated to the terminating party so he can accept or deny the reverse charge communication request.

Editor's Note: There is no stage 3 description for Reverse Charging in this Release: FFS.

#### 5.1.1.21	AoC charging

The Advice of Charge (AoC) service allows the served user to be provided with MMTel service charging information related to:

-	AoC at communication set-up time (AoC-S);

-	AoC during the communication (AoC-D);

-	AoC at the end of the communication (AoC-E).

### 5.1.2	Supplementary services management by the user

When an IMS supplementary service control can be provided to the UE as a subscription option, every action performed by the user for this service (Provision/Withdrawal, Registration/Erasure, Activation/Deactivation, interrogation) may be subject to produce charging information, but this aspect is out of the scope of the present document.

### 5.1.3	OCS-provided announcements for MMTel service

During an online charging session from an AS providing MMTel service and supplementary services, the OCS may utilize the Announcement service specified in TS 32.281 [41] to request this AS to render video or audio announcements to a subscriber involved in the MMTel service/supplementary services.

Further details on announcements interaction with MMTel supplementary services are specified in clause 5.3.2.1

### 5.1.4	CHF provided announcements for MMTel service

During converged charging sessions from an AS providing MMTel service and supplementary services, the CHF may utilize the Announcement service specified in TS 32.281 [41] to request this AS to render video or audio announcements to a subscriber involved in the MMTel service/supplementary services.

Further details on announcements interaction with MMTel supplementary services are specified in clause 5.4.2.

## 5.2	MMTel offline charging scenarios

# 5.2.1	Basic principles

The MMTel offline charging functionality is based on the CTFs reporting accounting information, by sending Diameter Charging Data Requests Charging Data Request [start, interim, stop and event] to the CDF.

The circumstances on which the Diameter client uses  Charging Data Request[start, interim and stop], or Charging Data Request[Event] depend on the supplementary service type and is determined for each of them. Further details are specified in clause 5.2.2.

These Diameter Charging Data Request triggers may be configured in such a way several MMTel supplementary services can be regrouped. Providing this flexibility allows to improve situations where several MMTel supplementary services are handled within the same AS for complying with interactions requirements associated to these MMTel supplementary services.

### 5.2.2	Diameter message flows

#### 5.2.2.0 	Introduction

The flows described in the present document specify the charging communications between the different CTF entities and the charging functions for different charging scenarios. The SIP messages and Diameter transactions associated with these charging scenarios are shown primarily for general information and to illustrate the charging triggers. 
They are not intended to be exhaustive of all the SIP message flows discussed in TS 24.228 [221] and they depend on the Diameter Charging Data Requests triggers configured by the operator.

Although each MMTel  supplementary service is described by separated flows illustrating the dedicated trigger(s) for this MMTel supplementary service, the Diameter Charging Data Request triggers (as stated in 5.2.1), may be configured with several MMtel supplementary services information.

#### 5.2.2.1	Message flows - Successful cases and scenarios

##### 5.2.2.1.0	Introduction

Following message flows are defined in TS 32.260 [20], and can be re-used for charging the basic multimedia telephony capabilities:

-	Session Establishment -  IMS Origination;

-	Session Establishment-  IMS Termination;

-	Mid-Session Procedures;

-	Session Release - Mobile Initiated.

##### 5.2.2.1.1	Originating Identification Presentation (OIP) charging

Figure 5.2.2.1.1.1 shows the Diameter transactions that are required between AS and CDF, which implements the OIP service, and CDF after service execution.

# Figure 5.2.2.1.1.1: Message sequence chart for offline charging of OIP service

##### 5.2.2.1.2	Originating Identification Restriction (OIR) charging

Figure 5.2.2.1.2.1 shows the Diameter transactions that are required between AS and CDF, which implements the OIR service, and CDF after service execution.

# Figure 5.2.2.1.2.1: Message sequence chart for offline charging of OIR service

##### 5.2.2.1.3	Terminating Identification Presentation (TIP) charging

Figure 5.2.2.1.3.1 shows the Diameter transactions that are required between AS and CDF, which implements the TIP service, and CDF after service execution.

# Figure 5.2.2.1.3.1: Message sequence chart for offline charging of TIP service

##### 5.2.2.1.4	Terminating Identification Restriction (TIR) charging

Figure 5.2.2.1.4.1 shows the Diameter transactions that are required between AS and CDF, which implements the TIR service, and CDF after service execution.

# Figure 5.2.2.1.4.1: Message sequence chart for offline charging of TIR service

##### 5.2.2.1.5	Communication Hold (HOLD) charging

Figure 5.2.2.1.5.1 shows the Diameter transactions that are required between AS which implements the HOLD service, and CDF after service execution. The involvement of the AS is optional as it is involved to the HOLD service provision only for announcement purposes.

# Figure 5.2.2.1.5.1: Message sequence chart for offline charging of HOLD service

# NOTE: 	Based on TS 24.610 [207] a scenarion triggerted by Re-Invite is also possible.

##### 5.2.2.1.6	Communication Barring – CB (ICB/ACB) charging

###### 5.2.2.1.6.1	Communication Barring (CB) – ICB and Charging Data Request

Figure 5.2.2.1.6.1.1 shows the Diameter transactions that are required between AS and CDF, which implements the CB service, and CDF after service execution.

# Figure 5.2.2.1.6.1.1: Message sequence chart for offline charging of CB service

###### 5.2.2.1.6.2	Communication Barring (CB) – OCB

Figure 5.2.2.1.6.2.1 shows the Diameter transactions that are required between AS and CDF, which implements the CB service, and CDF after service execution.

# Figure 5.2.2.1.6.2.1: Message sequence chart for offline charging of CB service

##### 5.2.2.1.7	Communications Diversion (CDIV) charging

###### 5.2.2.1.7.1	Communications Diversion (CDIV) – successful establishment

Figure 5.2.2.1.7.1.1 shows the Diameter transactions that are required between AS,implementing  the CDIV service and CDF for a successful communication forwarding on no reply.

Figure 5.2.2.1.7.1.1 : Message sequence chart for offline charging of CDIV service - establishment

# A communication is requested towards User B, user B has activated the CFNR

1.	SIP INVITE request incoming for User B. Based on the Initial Filter Criteria (IFC) rules, indicating that User B is subscribed to the CDIV supplementary services, the communication is forwarded to an AS implementing AS implementing CDIV. Then SIP INVITE is forwarded to User B.

2.	180 ringing is sent back from User B.

3.	The non-reply timer in the AS is started .

4.	The timer expires.

5 to 7. The AS implementing the CDIV service performs the Call Forwarding No Reply logic: release the communication to User B, and forward the call towards user C (SIP INVITE request including URI-C as destination).

8.	The destination User C party answers and a final response is received.

9.	Upon reception of the final response, the AS implementing the CDIV service sends a Charging Data Request [Start/Event] to record call forwarding execution (start of the forwarded leg from User B to User C): basic communication information are transferred with specific call forwarding information (service mode= "CFNR", associated number = " user B" as the user who invoked the call forwarding).

10.	The CDF acknowledges the reception of the data and opens/creates an AS CDR to record the forwarded leg from User B to User C.

NOTE : 	Although only the "call forwarding on no reply" case is depicted here , it serves as a basis for all other call forwarding modes description  (Call Forwarding Unconditional, Call Deflection, Call Forwarding on Busy, Call Forwarding Not Logged-in ) for Charging Data Request generation : In all these cases, the AS AS implementing the CDIV service sends a Charging Data Request [Start]  to record call forwarding execution (whatever the mode), when the final response is received from user-C.

###### 5.2.2.1.7.2	Communications Diversion (CDIV) – release

Figure 5.2.2.1.7.2.1 shows the Diameter transactions occurring on release of the previous established communication, initiated by user C.

# Figure 5.2.2.1.7.2.1: Message sequence chart for offline charging of CDIV service -release

1.	User C initiates release of the communication.

2.	At session termination the AS implementing the CDIV service, sends a Charging Data Request [Stop]  to record stop the call for which User B has been forwarded. 
The AS CDR is closed.

##### 5.2.2.1.8	Communication Waiting (CW) charging

Figure 5.2.2.1.8.1 shows the Diameter transactions that are required between AS, which implements the CW service and CDF for the callee (subscriber of CW).

# Figure 5.2.2.1.8.1: Message sequence chart for offline charging of CW service

1-2.  The communication is initiated by UE-A by sending an SIP INVITE request. The Request URI includes the URI of UE-B. After IFC evaluation in the S-CSCF the SIP INVITE request is routed to the CW AS.

2a.  The AS detects the CW condition and inserts a CW indication into the SIP INVITE request per procedures.

3-4. The SIP INVITE is routed to UE-B.

5.  UE-B recognizes the CW indication per procedures.

6.  UE-B sends back a 180 (Ringing) response.

6a. Out of scope: user B uses the HOLD service or releases a session in order to free resources.

7-8. The SIP 180 (Ringing) response is routed back to the AS.

8a.  The AS optionally inserts a Alert-Info with a 'CW' URN into the 180 (Ringing) response.

9-10.The SIP 180 (Ringing) response is routed back to the communication origin.

9a. The AS may initiate an announcement to the calling user that the communication is a waiting communication, in accordance with TS 24.628 [4].]

11-14. UE-B sends back a SIP 200 (OK) response to the CW AS and CW AS sends it to the S-CSCF.

15-17. The CW AS sends a Charging Data Request [Start]/ Charging Data Request [Event] to CDF, then the CDF opens/creates an AS CDR for the Communication Waiting with CW indication on the AS CDR and returns Charging Data Response to the CW AS.

18. S-CSCF sends back a SIP 200(OK) response to UE-A.

##### 5.2.2.1.9	Explicit Communication transfer (ECT) charging

###### 5.2.2.1.9.1	Explicit Communication transfer (ECT): Blind Transfer

Figure 5.2.2.1.9.1.1 and figure 5.2.2.1.9.1.2 show the Diameter transactions that are required between ASs implementing the ECT service (for the transferor and for the transferee) and CDF: a successful Blind Transfert from User A to User C, initiated by User B.

For diagram simplification, only one CDF is shown.

Figure 5.2.2.1.9.1.1: Message sequence chart for offline charging of ECT service – 
Blind Transfer (part1)

Figure 5.2.2.1.9.1.2: Message sequence chart for offline charging of ECT service – 
Blind Transfert (part 2)

In this scenario User A is the transferee, User B is the transferor, and User C is the transfer target.

User A and User B are in an established communication for which, based on the Initial Filter Criteria (IFC) rules indicating that User B is subscribed to the ECT supplementary service, the SIP INVITE was forwarded to an AS implementing ECT (for User A and User B).

User B puts User A on Hold.

1.	User B sends SIP REFER request in the existing A-B dialog, to initiate transfer User A to User C.

2. 	this subsequent request is forwarded to the AS implementing ECT.

3. 	AS generates an "ECT Session Identifier" replacing User C as the new destination information, for remaining in the loop, for transferor's role.

4. 	On SIP REFER receipt, this request is forwarded the AS implementing ECT.

5.	The SIP REFER is accepted by User A, and 202 Accepted is transferred from User A to User B.

6-9. User B sends SIP BYE for terminating original SIP INVITE with User A, acknowledged by SIP 200 OK from User A. 
AS B releases the media between User A and User B which are on hold, then sends a Charging Data Request [Stop] to record termination of the dialogue between User A and User B.

10.  More signalling between (User A,x-CSCF,AS-A ) and (x-CSCF,AS-B,User B) : User A sends SIP NOTIFY(100 Trying) associated to the received REFER, and User B acknowledges by SIP 200 OK..

11.  User A initiates a new session by sending an SIP INVITE request with "ECT Session Identifier".

12.  AS implementing the ECT service correlates this SIP INVITE to the initial session to be transferred and replaces "ECT Session Identifier" with User C for creating an SIP INVITE towards UE-C.

13 	The destination User C party answers and a final response is received.

14.  Upon reception of the final response, the AS implementing the ECT service for the transferor, sends a Charging Data Request [Start] to record Transfer execution.

15.	The CDF acknowledges the reception of the data and opens/creates an AS CDR.

16.	Upon reception of the final response, the AS implementing the ECT service for the transferee, sends a Charging Data Request [Start] to record User A is transferred, with specific indicator of whether the transferee was engaged in an originating call or in a terminating call before the transfer execution.

17.	The CDF acknowledges the reception of the data and opens/creates an AS CDR.

NOTE 1: 	The "Consultative transfer" scenario mainly differs from the "Blind transfer" on the transfer establishment phase : when User B has put User A on Hold, User B establishes a communication towards User C and puts User C on Hold, then User B initiates the SIP REFER for the transfer triggering. The Charging Data Request [Start] for recording Transfer execution occurs at the same steps for both scenarios: on the final response associated to SIP INVITE towards User C for the transferor and for the transferee.

NOTE 2:	The "Assured transfer" scenario mainly differs from the "Blind transfer" on the transfer establishment phase: User B may include an Expires header field in the Refer-To URI of the SIP REFER request when initiating transfer.  If User B has not received a SIP NOTIFY request indicating the end of the transfer operation within the duration of the Expires header allow or received a SIP NOTIFY indicating that the Assured transfer attempt failed, User B may request to terminate the transfer attempt by sending a SIP REFER request in the original communications dialog. Then User B may decide to retrieve the original communication by sending a SIP re-INVITE message in the original SIP dialog. 
A Charging Data Request [Event]  when the termination SIP NOTIFY is send out. 
A Charging Data Request [Interim]  when the retrieving SIP re-INVITE is sent out.

###### 5.2.2.1.9.2	Explicit Communication transfer (ECT): Release

Figure 5.2.2.1.9.2.1 shows the Diameter transactions that are required between ASs implementing the ECT service (for the transferor and for the transferee) and CDF: release from User A, after a successful Blind Transfer from User A to User C, initiated by User B.

# Figure 5.2.2.1.9.2.1: Message sequence chart for offline charging of ECT service – Release

1. 	User A initiates release of the communication.

2-3. 	At session termination the AS implementing the ECT service for the transferee, sends a Charging Data Request [Stop]  to record stop the call for which User A has been transferred.

4-5. 	At session termination the AS implementing the ECT service for the transferor, sends a Charging Data Request [Stop]  to record stop the call transferred by User B. The AS CDR is closed.

##### 5.2.2.1.10	Message Waiting Indication (MWI) charging

Figure 5.2.2.1.10.1 shows the Diameter transactions that are required between AS (MWI), which implements the MWI service and CDF.

# Figure 5.2.2.1.10.1: Message sequence chart for offline charging of MWI service

1-6. 	UE subscribes to AS (MWI) and the AS (MWI) returns SIP 200 OK.

7-10. 	The AS (MWI) sends SIP NOTIFY request to S-CSCF and the subscriber UE acknowledges the SIP NOTIFY request.

11-13. 	The AS (MWI) sends Charging Data Request [Event] message to CDF, CDF creates an AS CDR and returns Charging Data Responseto the AS (MWI).

##### 5.2.2.1.11	Conderence (CONF) Charging

##### 5.2.2.1.11.0	Introduction

During a conference, the user could create the conference, join a conference, invite another user to the conference and leave the conference, according to TS 24.605 [203]. The following subclauses respectively show the Diameter transactions that are required between AS, which implements the CONF service and CDF corresponding different conference scenarios.

###### 5.2.2.1.11.1	CONF charging – user creating a conference

Figure 5.2.2.1.11.1.1 : Message sequence chart for offline charging of CONF service - 
User creating a conference

1-2.  The communication is initiated by UE#1 by sending an SIP INVITE request to S-CSCF. And S-CSCF sends back 100 Trying reponse to inform UE#1 to wait for a while because the request is being treated.

3-4.	S-CSCF transfers SIP INVITE request from UE#1 to CONF AS and the CONF AS sends back 100 Trying response to inform S-CSCF to wait for a while because the request is being treated.

5.	 The CONF AS allocates the conference URI.

6-16.	After the media resource negociation process, the CONF AS sends back SIP 200 OK  response.

17-19.	The CONF AS sends Charging Data Request [Start]/Charging Data Request [Event] to CDF, then the CDF opens/creates an AS CDR for the conference with CONF indication on the AS CDR and returns Charging Data Response to the CONF AS.

###### 5.2.2.1.11.2	CONF charging – user joining a conference

5.2.2.1.11.2.1 shows the Diameter transactions that are required between AS, which implements the CONF service and CDF corresponding to the conference scenario: user joining a conference.

# Figure 5.2.2.1.11.2.1: CONF charging – user joining a conference

1-2.	The conference has already been created. UE#1 sends an SIP INVITE request to S-CSCF in order to join in the conference. And S-CSCF sends back SIP 100 Trying reponse to inform UE#1 to wait for a while because the request is being treated.

3-4.	S-CSCF transfers SIP INVITE request from UE#1 to CONF AS and the CONF AS sends back SIP 100 Trying response to inform S-CSCF to wait for a while because the request is being treated.

5-15.	After the media resource negociation process, the CONF AS sends back SIP 200 OK response.

16-18.	The CONF AS sends Charging Data Request [Interim/Event] to CDF, CDF updates or creates an AS CDR for the conference and returns Charging Data Response to the CONF AS.

###### 5.2.2.1.11.3	CONF charging – user inviting another user to a conference

Figure: 5.2.2.1.11.3.1 shows the Diameter transactions that are required between AS, which implements the CONF service and CDF corresponding to the conference scenario: user being invited into a conference.

# Figure5.2.2.1.11.3.1: CONF charging – user inviting another user to a conference

1-2.	The conference has already been created. UE#1 sends a SIP REFER request to S-CSCF in order to invite UE#2 into the conference.

3-4.	The CONF AS sends back 202 Accepted response to UE#1 via some related NEs like S-CSCF to indicate that he has received the SIP REFER request successfully.

5-6.	The CONF AS sends a SIP NOTIFY request corresponding the SIP REFER request to UE#1.

7-8.	UE#1 sends back SIP 200 OK reponse to the CONF AS.

9-10.	The CONF AS sends an SIP INVITE request to UE#2 in order to invite him into the conference. And UE#2 sends back SIP 200 OK response to the CONF AS.

11-13. The CONF AS sends Charging Data Request [Interim/Event] to CDF, CDF updates or creates an AS CDR for   the conference and returns Charging Data Response to the CONF AS.

###### 5.2.2.1.11.4	CONF charging – user leaving a conference

Figure: 5.2.2.1.11.4.1 shows the Diameter transactions that are required between AS, which implements the CONF service and CDF corresponding to the conference scenario: user leaving a conference.

# Figure: 5.2.2.1.11.4.1: CONF charging – user leaving a conference

1-2.	The conference has already been created. UE#1 sends a SIP BYE request to the CONF AS in order to quit the conference.

3-5.	The CONF AS sends Charging Data Request [Interim/Event] to CDF, CDF updates or creates an AS CDR for the conference and renturns Charging Data Response to the CONF AS. Conference termination should refer to the description of clause 5.3.2.7 in TS 24.147 [220], e.g. if there is not any conference policy and the last online conference user quits the conference could be considered as the conference termination, when the last online conference user quits the conference (sends a SIP BYE request to the CONF AS), the CONF AS sends Charging Data Request [Stop/Event] to CDF, CDF stops or creates an AS CDR for the conference and returns Charging Data Response to the CONF AS.

6-7.	The CONF AS sends back SIP 200 OK response to UE#1.

8-10.	The CONF AS sends a SIP NOTIFY request to other conference participants that UE#1 has quitted the conference.

11-12.	Other conference participants send back SIP 200 OK response to the CONF AS.

###### 5.2.2.1.11.5	Three-Party (3PTY) charging – successful establishment

Figure 5.2.2.1.11.5.1 shows the Diameter transactions that are required between AS, which implements the 3PTY service and CDF corresponding to the 3PTY scenario: 3PTY service successful establishment.

AS (3PTY)

Figure 5.2.2.1.11.5.1: Three-Party (3PTY) charging – successful establishment

1. 	UE-A and UE-B has established communication. UE-A puts UE-B on hold before invoking the 3-Way Calling with UE-C.

2-4. 	UE-A establishes a call with UE-C following normal call setup procedure and gets UE-C's permission to start the 3-Way Calling.

5. 	UE-A sends an SIP INVITE request to the 3PTY AS to establish a 3PTY session.

6-7. 	The 3PTY AS sends a SIP 200 OK response and receives an ACK request from UE-A.

8-9. 	UE-A sends a SIP REFER request to UE-B with the Refer-To header set to the address of the 3PTY AS; UE-B accepts the SIP REFER request.

10-11. 	UE-B sends a SIP NOTIFY request to UE-A to indicate that UE-B is acting on the SIP REFER request.

12. 	UE-B sends an SIP INVITE request to the 3PTY AS to join the 3PTY session.

13-14.	The 3PTY AS sends a SIP 200 OK response to UE-B and receives an ACK request.

15-16. 	UE-B sends a SIP NOTIFY request to UE-A to indicate that it has finished action required by the SIP REFER request.

17-18.	UE-A sends a SIP BYE request to terminate the call between UE-A and UE-B.

19-20.	Repeats step 8 to 18 on UE-Aand UE-C to terminate the call between UE-A and UE-C.

21-23. 	3PTY AS sends an Charging Data Request [Start/Interim/Event] to CDF, CDF opens or updates or create an 3PTY AS CDR for the 3PTY service and returns Charging Data Response to the 3PTY AS.

###### 5.2.2.1.11.6	Three-Party (3PTY) charging – release

Figure 5.2.2.1.11.6.1 shows the Diameter transactions that are required between AS, which implements the 3PTY service and CDF corresponding to the 3PTY scenario: release the 3PTY service.

# Figure 5.2.2.1.11.6.1: Three-Party (3PTY) charging – release

1. 	The 3PTY has already been established. UE-A sends a SIP BYE request to the 3PTY AS in order to release from the 3PTY service.

2-4. 	At session termination 3PTY AS, sends an Accounting-Request [Stop/Event] to CDF, CDF stops or creates an AS CDR for the 3PTY service and returns Accounting Answer to the 3PTY AS.

5. 	The 3PTY AS sends back SIP 200 OK response to UE-A.

6. 	The 3PTY AS sends a SIP NOTIFY request to other participants that 3PTY service has released.

##### 5.2.2.1.12	CCBS charging

Figure 5.2.2.1.12.1 shows the Diameter transactions that are required between AS and CDF, which implements the CCBS service, and CDF after service execution.

# Figure 5.2.2.1.12.1 : Message sequence chart for offline charging of CCBS service

1-5)	The communication is initiated by UE-A by sending an SIP INVITE request.

6)	UE-B answers with a SIP 486 (Busy Here) response. The SIP 486 (Busy Here) response is routed back to the terminating AS.

7-8)	The terminating AS inserts a Call-Info header field in the SIP 486 (Busy Here) response. The Call-Info header field contains the URI of the terminating AS with an "m" header field parameter set to "BS" (busy subscriber). It further includes a "purpose" header field parameters set to "call-completion". The SIP 486 (Busy Here) response is routed back to the originating AS.

9-10)	The originating AS sends back a SIP 183 (Session Progress) response to UE-A and initiates IVR procedures. 
User A is informed that CCBS is possible. User A activates CCBS.

11-14)	The originating AS subscribes for the call-completion event package. The terminating AS accepts the subscription and starts busy state supervision procedures on the callee.

15-17)	The terminating AS sends a notification to the originating AS.

18-20)	The originating AS sends Charging Data Request [Event] to CDF, then the CDF creates an AS CDR for the CCBS service subscriber with CCBS indication on the AS CDR and returns Charging Data Response to the originating AS.

21)	After confirmation of the notification the originating AS starts announcements procedures informing about the activation of CCBS.

22-23)	The originating AS forwards the SIP 486 (Busy Here) response to UE-A.

24-27)	When UE-B becomes available, the terminating AS sends a SIP NOTIFY request to the originating AS,

28-29)	The originating AS starts the CCBS recall by sending an SIP INVITE request to UE-B. In order to mark the SIP INVITE request as a prioritized request for call-completion, the originating AS adds the "m" SIP URI parameter with the value 'BS' to the Request-URI.

##### 5.2.2.1.13	CCNR charging

Figure 5.2.2.1.13.1 shows the Diameter transactions that are required between AS and CDF, which implements the CCNR service, and CDF after service execution.

# Figure 5.2.2.1.13.1: Message sequence chart for offline charging of CCNR service

1-5.	The communication is initiated by UE-A by sending an SIP INVITE request.

6.	UE-B answers with a 180 (Ringing) response. The 180 (Ringing) response is routed back to the terminating AS.

7-8.	The terminating AS inserts a Call-Info header field in the 180 (Ringing) response. The Call-Info header field will contain the URI of the terminating AS with a "m" header field parameter set to "NR" (no reply). It further includes a "purpose" header field parameter set to "call-completion". The 180 (Ringing) response is routed back to the originating AS.

9-10.	The originating AS removes the Call-Info header field, forwards the 180 (Ringing) response to UE-A and initiates IVR procedures. User A is informed that CCNR is possible. User A activates CCNR.

11-14.	The originating AS subscribes for the call-completion event package. The terminating AS accepts the subscription and starts activity supervision procedures on the callee.

15-17.	The terminating AS sends a notification to the originating AS,.

18-20.	The originating AS sends Charging Data Request [Event] to CDF, then the CDF creates an AS CDR for the CCNR service subscriber with CCNR indication on the AS CDR and returns Charging Data Response to the originating AS.

21.	After confirmation of the notification the originating AS starts announcements procedures informing about the activation of CCNR.

##### 5.2.2.1.14	Flexible Alerting (FA)

###### 5.2.2.1.14.1	Flexible Alerting (FA) - establishment

Figure 5.2.2.1.14.1.1 shows the Diameter transactions that are required between AS, implementing the FA service and CDF, with answer from one of FA group members.

# Figure 5.2.2.1.14.1.1: Message sequence chart for offline charging of FA service - Answered

UE-B and UE-C are FA group members of the FA group identified by the "Pilot Identity".

A communication is requested from UE-A towards the "Pilot Identity".

1. 	SIP INVITE request incoming for "Pilot Identity". Based on the Initial Filter Criteria (IFC), indicating the request must be forwarded to an AS implementing FA for this "Pilot Identiry".

2.	The AS implementing FA maps the Pilot Identity to the list of  FA group members: UE-B and UE-C, and  forwards SIP INVITE to all FA members.

3 to 4.	SIP INVITE forwarded towards UE-B and alerting phase signalling occurs with UE-A.

5 to 6. SIP INVITE forwarded towards UE-C and alerting phase signalling occurs with UE-A.

7.	UE-C answers.

8 to 10. 	Upon reception of the SIP 200 OK answer from UE-C, AS implementing the FA service:

-	Sends  Charging Data Request [Start] with FA MMTel supplementary service indication. 
The CDF creates an AS CDR and returns Charging Data Response to the AS.

-	Sends SIP CANCEL to other FA group member being alerted, UE-B.

###### 5.2.2.1.14.2	Flexible Alerting (FA) – call release

Figure 5.2.2.1.14.2.1 shows the Diameter transactions that are required between AS, implementing the FA service and CDF, when previous successfully established call is released.

# Figure 5.2.2.1.14.2.1: Message sequence chart for offline charging of FA service - Release

1. 	UEC initiates release of the communication.

2 to 3. 	At session termination the AS implementing the FA service, sends  Charging Data Request [Stop] and the AS CDR is closed.

##### 5.2.2.1.15	Malicious Communication Identification (MCID)

Figure 5.2.2.1.15.1 shows the Diameter transactions that are required between AS implementing the MCID service and CDF for a successful MCID delivery for permanent mode or temporary mode.

Figure 5.2.2.1.15.1: Message sequence chart for offline charging of MCID service

A communication is requested towards User B,  and User B  is subscribed to MCID service.

1. 	SIP INVITE request incoming for User B. Based on the Initial Filter Criteria (IFC) rules, indicating that User B is subscribed to the MCID supplementary services, the communication is forwarded to an AS implementing MCID.

2-4. 	In case of MCID temporary mode, AS stores relevant data temporarily, in case permanent mode AS registers the data and sends Accounting-Request [Event]  to record invocation of MCID, acknowledged by the CDF when AS CDR is created.Then SIP INVITE is forwarded to User B.

5.  	SIP 200 OK answer received from User B.

6. 	Further SIP signalling for communication establishment, or mid-communication take place, before SIP BYE from User-B.

7-10.	Upon reception of SIP re-INVITE from User-B, indicating temporary MCID request, AS registers the data previously stored and sends Charging Data Request[Event/interim] message to CDF  to record invocation of MCID, the CDF creates/updates  AS CDR and returns Charging Data Response.

##### 5.2.2.1.16	Customized Alerting Tone (CAT)

Although CAT Supplementary services may be delivered according to different models (CAT forking, CAT early session, CAT Gateway) as described in TS 24.182 [217], only one scenario is depicted here, and serves as a basis for CAT charging description, as the same principle applies.

Figure 5.2.2.1.16.1 shows the Diameter transactions that are required between AS implementing the CAT service and CDF for a successful communication establishment with CAT delivery.

More SIP signalling

Figure 5.2.2.1.16.1: Message sequence chart for offline charging of CAT service delivery

A communication is requested towards User B, user B has subscribed to CAT service.

1. 	SIP INVITE request incoming for User B. Based on the Initial Filter Criteria (IFC) rules, indicating that User B is subscribed to the CAT supplementary service, the communication is forwarded to an AS implementing CAT.

2.	The AS proceeds to CAT resources reservation and forwards SIP INVITE towards User B..

3.	180 ringing is sent back from User B.

4.	The AS sends a reliable 183 (session progress) response to User A with codecs to be used for CAT., acknowledged by PRACK from User A.

5 to 7. 	Upon acknowledgment from User A (SIP PRACK), the AS starts CAT media playing  (alerting tone), and sends  SIP 200 OK response to SIP PRACK towards User A.

- 8 to 11. Upon SIP 200 OK answer received from User B, AS stops CAT media playing (alerting tone), and sends a Charging Data Request [Start/Event] message to CDF,  CDF opens/creates an AS CDR and returns Charging Data Response.

NOTE:	The same following principle applies for all others CAT delivery scenarii: 
a Charging Data Request [Start] is sent from AS to record beginning of alerting tone playing, and Charging Data Request[Stop] is sent to record end of alerting tone playing.

##### 5.2.2.1.17	Closed User Group (CUG)

###### 5.2.2.1.17.1	Closed User Group (CUG): Originating

Figure 5.2.2.1.17.1.1 shows the Diameter transactions that are required between AS implementing the CUG service and CDF for a successful originating communication from User-A CUG member.

Figure 5.2.2.1.17.1.1: Message sequence chart for offline charging of CUG service - originating call

User-A is subscribed to CUG service. and initiates a communication towards User B.

1.  User-A initiates a communication towards User B sending SIP INVITE request. Based on the Initial Filter Criteria (IFC) rules, indicating that User A is subscribed to the CUG supplementary service, the communication is forwarded to an AS implementing CUG.

2. AS performs checks from the request received regarding User A CUG subscription profile, and determines the CUG-communication is allowed for CUG member User-A towards User-B : AS  forwards SIP INVITE towards User-B with CUG information included.

NOTE:	In case the result from AS checks indicates non-CUG communication behaviour (for CUG communication with outgoing access…), CUG information is not included, and non-CUG communication charging applies.

3. 	SIP signalling for communication establishment.

4-6. 	SIP 200 OK answer received from User B, and AS sends Charging Data Request [Start/Event] message to CDF to record start of the CUG-communication for User A CUG member. The CDF opens/creates an AS CDR and returns Charging Data Response

###### 5.2.2.1.17.2	Closed User Group (CUG): Terminating

Figure 5.2.2.1.17.2.1 shows the Diameter transactions that are required between AS implementing the CUG service and CDF for a successful terminating communication towards User-B CUG member.

Figure 5.2.2.1.17.2.1: Message sequence chart for offline charging of CUG service - terminating call

User-B is subscribed to CUG service.

1. 	incoming SIP INVITE request towards User-B. Based on the Initial Filter Criteria (IFC) rules, indicating that User B is subscribed to the CUG supplementary services, the communication is forwarded to an AS implementing CUG.

2. AS performs checks from the request received regarding User B CUG subscription profile, and determines the CUG-communication is allowed for CUG member User-B from User A: AS forwards SIP INVITE towards User-B.

NOTE: 	In case the result from AS checks indicates non-CUG communication behaviour (incoming access from non-CUG member…), non-CUG communication charging applies.

3. 	SIP signalling for communication establishment.

4-6. 	SIP 200 OK answer received from User B, and AS sends Charging Data Request [Start/Event] message to CDF to record start of the CUG-communication for User B CUG member. The CDF opens/creates an AS CDR and returns Charging Data Response.

##### 5.2.2.1.18	Personal Network Management (PNM)

Figure 5.2.2.1.18.1 shows the Diameter transactions that are required between AS implementing the PNM service and CDF for a successful "PN UE redirection" for a terminating communication, as described in TS 23.259 [218].

.

Figure 5.2.2.1.18.1: Message sequence chart for offline charging of "PNM-redirection" service

1. 	Incoming SIP INVITE request for UE-1 of User B. Based on the Initial Filter Criteria (IFC) rules, for UE-1, the communication is forwarded to the AS implementing PNM. From PN-user's PN configuration, the PNM AS determines the initial request is to be redirected to the default UE of the PN, i.e. to the UE-2.

2.	SIP INVITE is forwarded towards UE-2.

3.	Based on the Initial Filter Criteria (IFC) rules, for UE-2, the communication is forwarded to the AS implementing PNM.

4.	From PN-user's PN configuration, the PNM AS determines the initial request is to be sent to the default UE of the PN, i.e. to the UE-2. SIP INVITE is forwarded towards UE-2.

5.	The destination UE-2 party answers and a final response is received.

6.	Upon reception of the final response, the AS implementing the PNM service sends Charging Data Request [Start] to record "PN UE redirection" execution to CDF.

7.	The CDF acknowledges the reception of the data and opens/creates an AS CDR.

##### 5.2.2.1.19	Customized Ringing Signal (CRS)

Although CRS Supplementary services may be delivered according to different models (download and play model, DTMF...) as described in TS 24.183 [219], only one scenario is depicted here, and serves as a basis for CRS charging description, as the same principle applies.

Figure 5.2.2.1.19.1 shows the Diameter transactions that are required between AS implementing the CRS service and CDF for a successful communication establishment with CRS delivery.

Figure 5.2.2.1.19.1: Message sequence chart for offline charging of "CRS" service delivery

A communication is requested towards User B, user B has subscribed to CRS service.

1. 	SIP INVITE request incoming for User B. Based on the Initial Filter Criteria (IFC) rules, indicating that User B is subscribed to the CRS supplementary service, the communication is forwarded to an AS implementing CRS. The INVITE is forwarded towards User B.

2.	SIP 180 ringing is sent back from User B.

3 to 4. 	Upon acknowledgment from User A (SIP PRACK), the AS reserves CRS media, and forwards PRACK towards User B.

5-6.  Upon SIP 200 OK(SIP PRACK acknowledgment) from User B, the AS starts CRS media playing, and forwards SIP 200 OK towards User A.

7-10. Upon SIP 200 OK answer received from User B, the AS stops CRS media playing, and sends Charging Data Request [Start/Event] message to CDF, CDF opens/creates an AS CDR and returns Charging Data Response. The AS forwards SIP 200 OK towards User A.

NOTE: 	The same following principle applies for all others CRS delivery scenarii: 
a Charging Data Request [Start/Event] is sent from AS to record CRS service delivery.

##### 5.2.2.1.20	Advice of Charge (AoC)

The information flows showing the transactions between AS implementing the AoC service and CDF for AoC-S, AoC-D and AoC-E are described in TS 32.280 [69].

### 5.2.3	GTP' record transfer flows

As in IMS offline charging, GTP' is not used between CDF and CGF for MMTel offline charging, because CDF and CGF are combined into CCF (see clause 4.2).

NOTE:	Vendors may nevertheless implement a separate CDF and CGF for MMTel offline charging. 
In this case, it is recommended that the approach chosen conforms to the principles and protocol applications specified in TS 32.295 [54].

### 5.2.4	BI CDR file transfer

For further details on the Bi protocol application refer to TS 32.297 [52].

## 5.3	MMTel online charging scenarios

### 5.3.1	Basic principles

Online charging of MMTel services is done according to the general principles of Diameter Credit-Control Applications (Debit / Reserve Units Response) as specified in TS 32.299 [50].

The CTFs implementing the MMTel online charging functionality may apply:

-	Immediate Event Charging (IEC) with Debit Units Request [Event] generated, or

-	Event Charging with Unit Reservation (ECUR), or Session Charging with Unit Reservation (SCUR), both with Reserve Units Request [Initial or Termination] generated.

The circumstances on which IEC, ECUR or SCUR are applied, depend on the MMTel supplementary service and/or operator's policy. Further details are specified in clause 5.3.2.

### 5.3.2	Diameter message flows

#### 5.3.2.0	Diameter message flows

The flows described in the present document specify the charging communications between the different CTF entities and the charging functions for different online charging scenarios. The SIP messages and Diameter transactions associated with these online charging scenarios are shown primarily for general information and to illustrate the charging triggers. They are not intended to be exhaustive of all the SIP message flows discussed in TS 24.228 [221] and they depend on the Diameter Debit / Reserve Units Request triggers configured by the operator.

Each MMTel supplementary service is described by specific flows illustrating the dedicated trigger(s) for this MMTel supplementary service.

Following message flows are defined in TS 32.260 [20], and can be re-used for charging the basic multimedia telephony capabilities:

-	Successful Session Establishment;

-	Successful Session Establishment with Early Media Negotiation;

-	Mid-Session Procedures;

-	Session Release.

For OCS-provided announcement service during online charging session from the AS providing MMTel service for Session charging with Unit Reservation (SCUR), the message flows defined in TS 32.281 [41] apply.

#### 5.3.2.1	Message flows - Successful cases and scenarios

##### 5.3.2.1.0	Interaction with IMS-GWF

As an MMTel principle, when online charging has to be applied for an MMTel supplementary service, and the trigger list includes IMS-GWF together with MMTel AS (implementing the MMTel supplementary service), filter criteria should be configured to have the MMTel AS triggered before IMS-GWF, in order to prevent improper charging.

Figure 5.3.2.1.0.1 shows the Diameter transactions that are required between MMTel AS, IMS-GWF and OCS, when online charging is applied for MMTel supplementary service.

# Figure 5.3.2.1.0.1: Message sequence chart for online charging – IMS-GWF, MMTel AS and OCS

1. 	User-A initiates a session by sending SIP INVITE request. Based on the Initial Filter Criteria (IFC) rules, indicating that User A is subscribed to the MMTel supplementary service X, the communication is forwarded to an AS implementing this service.

2-3. 	As "online charging" is activated for User A, the AS sends a  Reserve Units Request [Initial, service X, ICID] to the OCS for requesting units for the X supplementary service. The OCS grants units in the  Reserve Units Response response for this request. 
The AS forwards SIP INVITE via the S-CSCF.

4. As "online charging" is activated for User A, IMS-GWF is triggered by the S-CSCF, and IMS-GWF sends a  Reserve Units Request [Initial, ICID] to the OCS for requesting units for the session identified by ICID.

5-6. The OCS detects this request is within a session for which there is already an ongoing online Credit-Control Diameter session (same ICID). Based on operator policy the OCS can use either Credit_Control_not_applicable to supersede online charging in S-CSCF or grant appropriate service units (GSU).

##### 5.3.2.1.1	Communications Diversion (CDIV)

###### 5.3.2.1.1.1	Communications Diversion (CDIV) – successful establishment

Figure 5.3.2.1.1.1.1 shows the Diameter transactions that are required between AS implementing the CDIV service and OCS, when online charging is applied to a successful CFU communication.

# Figure 5.3.2.1.1.1.1: Message sequence chart for online charging of CDIV service - establishment

A communication is requested towards User B: CFU and online-based charging activated for user B.

1. 	SIP INVITE request incoming for User B. Based on the iFC, indicating that User B is subscribed to the CDIV supplementary service, the communication is forwarded to AS implementing CDIV, where CFU is detected.

2.	As the subscriber is "online charging", the AS sends a Reserve Units Request [Initial, service CDIV, ICID..] to the OCS for requesting units for the CDIV (CFU) supplementary service.

3.	The OCS grants units in the Reserve Units Response response for this request.

4. 	The CDIV (CFU) can now be delivered: a SIP INVITE is sent towards user C via the S-CSCF. Any potential announcements to be played to the served party, provided by the OCS within the Reserve Units Response [Initial] , shall be discarded due to CDIV.

5-6. 	On answer from User-C (SIP 200 OK), the AS sends a  Reserve Units Request [Update, USU] with possible already used units and requests new units to continue.

7. 	New units are granted via Reserve Units Response and the SIP 200 OK is propagated. Any potential announcements to be played to the served party, provided by the OCS within the Reserve Units Response, shall be discarded due to CDIV.

###### 5.3.2.1.1.2	Communications Diversion (CDIV) – release

Figure 5.3.2.1.1.2.1 shows the Diameter transactions occurring on release of the previous established communication, initiated by user C:

Figure 5.3.2.1.1.2.1: Message sequence chart for offline charging of CDIV service -release

1. 	User C initiates release of the communication.

2. 	At session termination the AS implementing the CDIV service, sends a  Debit Units Request [Terminate, used service units] for ending Credit-Control.

##### 5.3.2.1.2	Flexible Alerting (FA)

Figure 5.3.2.1.2.1 shows the Diameter transactions that are required between AS implementing the FA service and OCS, when online charging is applied to a successful FA communication.

Figure 5.3.2.1.2.1: Message sequence chart for online charging of FA service

UE-B and UE-C are FA group members of the FA group identified by the "Pilot Identity".

A communication is requested from UE-A towards the "Pilot Identity".

1. 	SIP INVITE request incoming for "Pilot Identity". Based on the Initial Filter Criteria (IFC), indicating the request must be forwarded to an AS implementing FA for this "Pilot Identity".

2. 	As "online charging" is activated for the "Pilot Identity", the AS implementing FA has to initiate parallel SIP INVITE towards each of FA group members.

3. 	For FA member User B, the AS sends a  Reserve Units Request [Initial, service FA, ICID-B] to the OCS for requesting units for the FA supplementary service.

4. 	The OCS grants units in the Reserve Units Response response for this request.

5. 	SIP INVITE is sent towards User B via the S-CSCF.

6. 	Alerting phase signalling occurs with UE-A.

7. 	A parallel (from step 3 to step 6) scenario occurs towards User-C with session identified by ICID-C.

8-9. 	Upon reception of the SIP 200 OK answer from UE-C, AS sends SIP CANCEL to other FA group member being alerted (UE-B).

10. 	More signalling for SIP CANCEL transactions.

11-13. 	When the last SIP CANCEL transaction is terminated, AS sends a Reserve Units Request [Update, USU] with possible already used units, and requests new units to continue the communication between User A and User C.

14. 	New units are granted via Reserve Units Response.

15. 	SIP 200 OK is propagated towards User A.

During AS FA supplementary service execution and interaction with OCS, announcements to be played to the served party may be provided by the OCS within Reserve Units Responses [Initial/Update]. Announcement procedures handling by the AS are those specified in TS 32.281 [41].

##### 5.3.2.1.3	Closed User Group (CUG)

###### 5.3.2.1.3.1	Closed User Group (CUG): Originating

Figure 5.3.2.1.3.1.1 shows the Diameter transactions that are required between AS implementing the CUG service and OCS, when online charging is applied to a successful originating communication from User-A CUG member.

Figure 5.3.2.1.3.1.1: Message sequence chart for online charging of CUG service - originating call

User-A is subscribed to CUG service, and initiates a communication towards User B.

1. 	User-A initiates a communication towards User B sending SIP INVITE request. Based on the Initial Filter Criteria (IFC) rules, indicating that User A is subscribed to the CUG supplementary service, the communication is forwarded to an AS implementing CUG.

2. 	AS performs checks from the request received regarding User A CUG subscription profile, and determines the CUG-communication is allowed for CUG member User-A towards User-B.

3-4. 	As "online charging" is activated for User A, the AS sends a Reserve Units Request [Initial, service CUG, ICID] to the OCS for requesting units for the CUG supplementary service. The OCS grants units in the Reserve Units Response response for this request. 
The AS forwards SIP INVITE via the S-CSCF towards User-B with CUG information included.

NOTE: 	In case the result from AS checks indicates non-CUG communication behaviour (for CUG communication with outgoing access…), non-CUG communication charging applies, and CUG information is not included in the forwarded SIP INVITE.

5. 	SIP signalling for communication establishment.

6-8. 	SIP 200 OK answer received from User B, and AS sends a  Reserve Units Request [Update, USU] with possible already used units, and requests new units to continue the CUG communication between User A and User B. New units are granted via Reserve Units Response, and SIP 200 OK is propagated towards User A.

During AS CUG supplementary service execution and interaction with OCS, announcements to be played to the served party may be provided by the OCS within Reserve Units Responses [Initial/Update]. Announcement procedures handling by the AS are those specified in TS 32.281 [41].

###### 5.3.2.1.3.2	Closed User Group (CUG): Terminating

Figure 5.3.2.1.3.2.1 shows the Diameter transactions that are required between AS implementing the CUG service and OCS, when online charging is applied to a successful terminating communication towards User-B CUG member.

Figure 5.3.2.1.3.2.1: Message sequence chart for online charging of CUG service - terminating call

User-B is subscribed to CUG service.

1. 	Incoming SIP INVITE request with CUG information towards User-B.Based on the Initial Filter Criteria (IFC) rules, indicating that User B is subscribed to the CUG supplementary services, the communication is forwarded to an AS implementing CUG.

2. 	AS performs checks from the request received regarding User B CUG subscription profile, and determines the CUG-communication is allowed for CUG member User-B from User A.

3-4. 	As "online charging" is activated for User B, the AS sends a  Reserve Units Request [Initial, service CUG, ICID] to the OCS for requesting units for the CUG supplementary service.The OCS grants units in the Reserve Units Response response for this request. The AS forwards SIP INVITE via the S-CSCF towards User-B.

NOTE: 	In case the result from AS checks indicates non-CUG communication behaviour (incoming access from non-CUG member…), non-CUG communication charging applies.

5. 	SIP signalling for communication establishment.

6-8. 	SIP 200 OK answer received from User B, and AS sends a  Reserve Units Request [Update, USU] with possible already used units, and requests new units to continue the CUG communication between User B and User A. New units are granted via Reserve Units Response, and SIP 200 OK is propagated towards User A.

During AS CUG supplementary service execution and interaction with OCS, announcements to be played to the served party may be provided by the OCS within Reserve Units Responses [Initial/Update]. Announcement procedures handling by the AS are those specified in TS 32.281 [41].

##### 5.3.2.1.4	Conference (CONF)

###### 5.3.2.1.4.0	Introduction

Figure 5.3.2.1.4.1.1 to figure 5.3.2.1.4.5.1 show the Diameter transactions that are required between AS implementing the CONF service and OCS, when online charging is applied to different conference scenarios.

For CONF service, two different set of scenarios are provided for illustrating the situation where ECUR or SCUR apply, depending on operator's policy.

###### 5.3.2.1.4.1	CONF – user creating a conference - ECUR mode

Figure 5.3.2.1.4.1.1: Message sequence chart for online charging of CONF service- User creating a conference - ECUR mode

1.	User-A initiates a conference by sending an SIP INVITE request to S-CSCF. Based on the Initial Filter Criteria (IFC) rules, indicating that User A is subscribed to the CONF supplementary service, the communication is forwarded to an AS implementing CONF.

2.	The CONF AS allocates the conference resource.

3-4. 	As "online charging" is activated, the AS sends a  Reserve Units Request [Initial, service CONF, creation] to the OCS for requesting units for the CONF supplementary service creation. 
The OCS grants units in the Reserve Units Response response for this request.

5. 	Media resource negociation process.

6-7. 	The AS (CONF) sends a SIP 200 OK response replied by SIP ACK from UE-A.

8-9. 	On successful service delivery, AS (CONF) sends  Reserve Units Request [Terminate, USU] for ending Credit-Control, with granted units used for CONF creation.

###### 5.3.2.1.4.2	CONF – user creating a conference - SCUR mode

Figure 5.3.2.1.4.2.1: Message sequence chart for online charging of CONF service- User creating a conference - SCUR mode

1.	User-A initiates a conference by sending an SIP INVITE request to S-CSCF. Based on the Initial Filter Criteria (IFC) rules, indicating that User A is subscribed to the CONF supplementary service, the communication is forwarded to an AS implementing CONF.

2.	The CONF AS allocates the conference resource.

3-4. 	As "online charging" is activated, the AS sends a Reserve Units Request [Initial, service CONF, creation] to the OCS for requesting units for the CONF supplementary service creation. 
The OCS grants units in the Reserve Units Response response for this request.

5. 	Media resource negociation process.

6-7. 	The AS (CONF) sends a SIP 200 OK response replied by ACK from UE-A.

8-9. 	As CONF charging is based on duration or other CONF parameters, from start of the service on step 7), new units are subsequently requested by sending  Reserve Units Request [Update, USU]with already used units, for continuing the service when new CONF parameters or values are provided.

During AS CONF supplementary service execution and interaction with OCS, announcements to be played to the served party may be provided by the OCS within Reserve Units Responses [Initial/Update]. Announcement procedures handling by the AS are those specified in TS 32.281 [41].

###### 5.3.2.1.4.3	CONF – user joining a conference (SCUR mode)

Figure 5.3.2.1.4.3.1: Message sequence chart for online charging of CONF service- User joining a conference - SCUR mode

1. 	The conference has already been created by User A. User-B sends an SIP INVITE request to S-CSCF in order to join in the conference identified by the conference URI. The communication is forwarded to AS addressed by conference URI.

2. 	Media resource negociation process for adding UE-B to the conference.

3-4. 	The AS (CONF) sends a SIP 200 OK response replied by SIP ACK from UE-B.

5-6. 	This step applies for SCUR mode, when "Nb of participants" is needed for the "online charging" running for this conference: the AS (CONF) sends a Reserve Units Request [Update, USU]) with possible already used units, for requesting new units with new number of participants. The OCS grants units in the Reserve Units Response response for this request.

The OCS may provide announcements to be played to the served party UE-A within Reserve Units Response [Update]. Announcement procedures handling by the AS are those specified in TS 32.281 [41].

5.3.2.1.4.4	CONF – user inviting another user to a conference (SCUR mode)

Figure 5.3.2.1.4.4.1: Message sequence chart for online charging of CONF service - user inviting another user to a conference – SCUR mode

1. 	The conference has already been created by User A. User A sends aSIP REFER request to S-CSCF with the conference URI in order to invite User B into the conference. The communication is forwarded to the AS addressed by conference URI.

2-4. 	Sequence between UE-A and AS (CONF) to proceed the SIP REFER (accept, SIP NOTIFY...).

5. 	User B is invited to the Conference by SIP INVITE from AS (CONF).

6. 	Media resource negociation process for adding UE-B to the conference.

7-8. 	UE-B sends SIP 200 OK response when he joins the conf, replied by ACK from AS (CONF).

9-10. 	This step applies for SCUR mode, when "Nb of participants" is needed for the "online charging" running for this conference: the AS (CONF) sends a Reserve Units Request [Update, USU] with possible already used units, for requesting new units with new number of participants. The OCS grants units in the Reserve Units Response response for this request.

The OCS may provide announcements to be played to the served party UE-A within Reserve Units Response [Update]. Announcement procedures handling by the AS are those specified in TS 32.281 [41].

5.3.2.1.4.5	CONF – user leaving a conference (SCUR mode)

Figure 5.3.2.1.4.5.1: Message sequence chart for online charging of CONF service - user leaving a conference SCUR mode

1. 	The conference has already been created. User B sends a SIP BYE request request to the AS (CONF) in order to quit the conference.The communication is forwarded to the AS addressed by conference URI.

2-3. 	This step applies for SCUR mode:

-	In case "Nb of participants" is needed for the "online charging" running for this conference, and UE-B leaving does not cause conference termination: the AS (CONF) sends a  Reserve Units Request [Update, USU] with possible already used units, for requesting new units with new number of participants. The OCS grants units in the Reserve Units Response response for this request.

-	In case UE-B leaving does cause conference termination, the AS(CONF) sends a Reserve Units Request [Terminate, USU] with  used units, for ending Credit-Control.

4. 	The AS (CONF) sends back SIP 200 OK response to UE-B.

5-6. 	The AS (CONF) sends a SIP NOTIFY request to UE-B for unsubscribe UE-B to conf event package, replied by SIP 200 OK from UE-B.

7. 	This step does not apply if conference has terminated: 
Other conference participants are Notified UE-B has quit the conference.

###### 5.3.2.1.4.6	CONF (3PTY) – successful establishment

Figure 5.3.2.1.4.6.1 shows the Diameter transactions that are required between AS implementing the CONF (3PTY) service and OCS, when online charging is applied to a successful 3PTY conference scenario.

Figure 5.3.2.1.4.6.1:  Message sequence chart for online charging of CONF (3PTY) service - establishment

1. 	UE-A and UE-B have established communication. UE-A puts UE-B on hold.Before invoking the 3-Way Calling UE-A establishes a communication with UE-C.

2. 	UE-A sends an SIP INVITE request to the AS (CONF) to establish a 3PTY session.

3-4. 	As "online charging" is activated, the AS (CONF) sends a  Reserve Units Request [Initial, service CONF (3PTY)] to the OCS for requesting units for the service.The OCS grants units in the Reserve Units Response response for this request.

5-6. 	The AS (CONF) sends a SIP 200 OK response replied by SIP ACK from UE-A.

7. 	UE-A sends a SIP REFER request to UE-B with the Refer-To header set to the address of the AS (CONF).

8. 	Sequence for 3PTY establishment:

-	SIP REFER sequence (accept, SIP NOTIFY...) between UE-B and UE-A.

-	SIP INVITE from UE-B sequence for joining

-	Original communication UE-A UE-B sequence termination

-	SIP REFER sequence (accept, SIP NOTIFY...) between UE-A and UE-C.

9-11. 	SIP INVITE from UE-C sequence for joining the CONF (3PTY)

12-13. 	In case ECUR mode applies to CONF (3PTY) Charging, on SIP ACK receipt, AS (CONF) sends Reserve Units Request [Terminate, USU] for ending Credit-Control, with granted units used for 3PTYcreation.

In case SCUR mode applies to CONF (3PTY) charging (charging to be based on duration or other parameters), from ACK receipt, new units are subsequently requested by sending Reserve Units Request [Update, USU] with already used units, for continuing the service when new CONF parameters or values have are provided.

14. 	Original communication UE-A UE-C sequence termination, occuring at any time from step 11.

During AS CONF (3PTY) supplementary service execution and interaction with OCS, announcements to be played to the served party may be provided by the OCS within Reserve Units Responses [Initial/Update]. Announcement procedures handling by the AS are those specified in TS 32.281 [41].

##### 5.3.2.1.5	Explicit Communication transfer (ECT)

###### 5.3.2.1.5.0	Introduction

Figure 5.3.2.1.5.1.1 and figure 5.3.2.1.5.1.2 show the Diameter transactions that are required between ASimplementing the ECT service and OCS, when online charging is applied to a successful Blind Transfert from User A to User C, initiated by User B.

For diagram simplification, only one OCS is shown.

###### 5.3.2.1.5.1	Explicit Communication transfer (ECT): Blind Transfer with sending SIP REFER

Figure 5.3.2.1.5.1.1: Message sequence chart for online charging of ECT service establishment – Blind Transfer (part 1)

Figure 5.3.2.1.5.1.2:  Message sequence chart for online charging of ECT service establishment – Blind Transfer (part 2)

In this scenario User A is the transferee, User B is the transferor, and User C is the transfer target.

User A and User B are in an established communication, User B puts User A on Hold.

1. 	User B sends SIP REFER request in the existing A-B dialog, to initiate transfer User A to User C. Based on the iFC, indicating that User B is subscribed to the ECT supplementary service, the communication is forwarded to AS implementing ECT.

2-3. 	As "online charging" is activated, the AS sends a Reserve Units Request [Initial, service ECT] to the OCS for requesting units for the ECT supplementary service. The OCS grants units in the Reserve Units Response response for this request. The AS generates an "ECT Session Identifier" replacing User C as the new destination information and forwards SIP REFER towards User A. Any potential announcements to be played to the transferor, provided by the OCS within the Reserve Units Response [Initial], and subsequent Reserve Units Responses [Update], shall be discarded due to ECT.

4-5. 	As "online charging" is activated for User A, the AS sends a Reserve Units Request [Initial, service ECT] to the OCS for requesting units for the ECT supplementary service for the transferee. The OCS grants units in the Reserve Units Response response for this request. The AS forwards SIP REFER towards User-A. Any potential announcements to be played to the transferee, provided by the OCS within the Reserve Units Response [Initial], shall be discarded due to ECT.

6. 	More signalling between (User A, x-CSCF, AS-A) and (x-CSCF, AS-B,User B) for SIP REFER acceptation sequence and terminating previous media between User A and User B.

7. 	User A initiates a new session by sending an SIP INVITE request with "ECT Session Identifier".

8-9. 	AS implementing the ECT service for the transferee correlates this SIP INVITE to the transfer being processed, and new units are requested by sending Reserve Units Request [Update, USU] with already used units, for continuing the service. The AS forwards SIP INVITE request. Any potential announcements to be played to the transferee may be provided by the OCS within Reserve Units Responses [Update]. Announcement procedures handling by the AS are those specified in TS 32.281 [41].

10-11. 	AS implementing the ECT service for the transferor correlates this SIP INVITE to the initial session to be transferred and new units are requested by sending Reserve Units Request [Update, USU] with already used units, for continuing the service, prior to forwarding SIP INVITE towards UE-C.

12. 	The destination User C party answers, and a final response is received.

13-14. 	On transfer execution, the AS sends a Reserve Units Request [Update, USU] with possible already used units and requests new units for transferor to continue.

15-16. 	On transfer execution, the AS sends a Reserve Units Request [Update, USU] with possible already used units and requests new units for transferee to continue.

NOTE 1: 	For "Consultative Transfer" scenario, the same steps apply for interactions with OCS for transferee and transferor: Reserve Units Request [Initial] on SIP REFER initiating, intermediate Reserve Units Request [Update] for more units on SIP INVITE for new session, and Reserve Units Request [Update] for more units on transfer execution when User C answers.

NOTE 2: 	For "Assured Transfer" scenario, the same steps apply for interactions with OCS for transferee and transferor:  Reserve Units Request [Initial] on SIP REFER initiating, intermediate  Reserve Units Request [Update] for more units on SIP INVITE for new session, and  Reserve Units Request [Update] for more units on transfer execution when User C answers.Besides,  Reserve Units Request [Terminate] on transferring timeout or failure, and  Reserve Units Request [Update] for more units on original dialog retrievemet.

###### 5.3.2.1.5.2	Explicit Communication transfer (ECT): Blind Transfer with 3PCC

Figure 5.3.2.1.5.2.1:  Message sequence chart for online charging of ECT service establishment – Blind Transfer with 3PCC

In this scenario User A is the transferee, User B is the transferor, and User C is the transfer target, and the User B's AS acts as a 3PCC (Third Party Call Control) initiating B2BUA:

User A and User B are in an established communication, User B puts User A on Hold.

1. 	User B sends SIP REFER request in the existing A-B dialog, to initiate transfer User A to User C. Based on the iFC, indicating that User B is subscribed to the ECT supplementary service, the communication is forwarded to AS implementing ECT in a 3PCC mode.

2-3. 	As "online charging" is activated, the AS sends a Reserve Units Request [Initial, service ECT] to the OCS for requesting units for the ECT supplementary service. The OCS grants units in the Reserve Units Response response for this request. Any potential announcements to be played to the transferor, provided by the OCS within the Reserve Units Response [Initial], and subsequent Reserve Units Responses [Update], shall be discarded due to ECT.

4-5. 	AS B performs the SIP REFER procedure with User B, and sends SIP INVITE towards User C.

6-8. 	On Provisionnal response from User C, the AS sends a Reserve Units Request [Update, USU] with possible already used units and requests new units for transferor to continue.

9. 	AS B sends SIP Re-INVITE towards User A on existing dialog. From User A side, the basic SIP RE-INVITE procedure applies (ECT service not invoked).

10-13. 	On SIP 200 OK response from User A, the AS sends a Reserve Units Request [Update, USU] with possible already used units and requests new units for transferor to continue, and sends ACK back to User A.

14-16. 	On SIP 200 OK response from User C, the AS sends a Reserve Units Request [Update, USU] with possible already used units and requests new units for transferred communication to continue.

###### 5.3.2.1.5.3	Explicit Communication transfer (ECT): Release

Figure 5.3.2.1.5.3.1 shows the Diameter transactions that are required between ASs implementing the ECT service for the transferor and for the transferee, and OCS:  release from User A, after a successful Blind Transfer from User A to User C, initiated by User B.

Figure 5.3.2.1.5.3.1: Message sequence chart for offline charging of ECT service – release

1. 	User A initiates release of the communication.

2-3. 	At session termination the AS implementing the ECT service for the transferee, sends a Reserve Units Request [Terminate, USU] with used units, for ending Credit-Control.

4-5. 	At session termination the AS implementing the ECT service for the transferor, sends a Reserve Units Request [Terminate, USU] with used units, for ending Credit-Control.

NOTE:	For "Blind Transfer with 3PCC" release scenario, only interactions between AS-B and OCS apply for ECT service.

#### 5.3.2.2	Message flows - error cases and scenarios

Error cases and scenarios for SIP related procedures and Diameter related procedures used for MMTel service and Supplementary services charging are defined in TS 32.260 [20].

## 5.4	MMTel converged charging scenarios

### 5.4.1	Basic principles

#### 5.4.1.1	General

Converged charging may be performed by the MMTel AS interacting with CHF using Nchf specified in TS 32.290 [57] and TS 32.291 [58]. To provide the data required for the management activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics etc.), the MMTel AS shall be able to perform converged charging for each of the MMTel supplementary service.

The CTFs implementing the MMTel converged charging functionality may apply:

-	Immediate Event Charging (IEC) with Charging Data Request [Event] generated, or

-	Session based charging SCUR, with Charging Data Request [Initial, Update, Termination] , or

-	Post Event Charging (PEC) with Charging Data Request [Event].

The circumstances on which IEC, SCUR or PEC are applicable, depend on the MMTel supplementary service and/or operator's policy.

Editor's note:	The use of ECUR is FFS.

Converged charging uses centralized or decentralized unit determination and centralized rating scenarios for event based convergent charging specified in TS 32.290 [57].

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [58].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [51].

The selection of the CHF can be configured in the MMTel AS.

Editor's note:	The CHF selection mechanism using other means, e.g. SIP and user profile, is FFS.

#### 5.4.1.2	Applicable Triggers for MMTel

##### 5.4.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. IMPI).

Each trigger condition (i.e. chargeable event) defined for the MMTel converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.4.1.2.1 summarizes the set of default trigger conditions, including their category and if they are possible to change, which shall be supported by the MMTel AS. For "immediate report" category, the table also provides the corresponding Charging Data Request [Initial, Udate, Termination] message sent towards the CHF.

Table 5.4.1.2.1: Default Trigger conditions for MMTel

| Trigger Conditions |  | Trigger level | Converged Charging default category | Offline only charging default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Originating Identification Presentation (OIP) |  |  |  |  |  |  |
| Invite OK |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Originating Identification Restriction (OIR) |  |  |  |  |  |  |
| Invite OK |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Terminating Identification Presentation (TIP) |  |  |  |  |  |  |
| Invite OK |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Terminating Identification Restriction (TIR) |  |  |  |  |  |  |
| Invite OK |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Communication Hold (HOLD) |  |  |  |  |  |  |
| Update OK |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Communication Barring (CB) |  |  |  |  |  |  |
| Invite |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Message Waiting Indication (MWI) |  |  |  |  |  |  |
| Notify |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Conference (CONF) |  |  |  |  |  |  |
| Invite |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Created |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| User joining |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| User leaving |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Bye |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Completion of Communication sessions to Busy Subscriber (CCBS) |  |  |  |  |  |  |
| Notify |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Completion of Communications by No Reply (CCNR) |  |  |  |  |  |  |
| Notify |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Communications Diversion (CDIV) |  |  |  |  |  |  |
| Invite (OK) |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Answer |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Bye |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Communication Waiting (CW) |  |  |  |  |  |  |
| Invite OK |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Explicit Communication Transfer (ECT) |  |  |  |  |  |  |
| Refer |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Invite |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Answer |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Release |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Flexible Alerting (FA) |  |  |  |  |  |  |
| Invite |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Answer/Cancel |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Bye |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Malicious Communication Identification (MCID) |  |  |  |  |  |  |
| Store |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Customized Alerting Tone (CAT) |  |  |  |  |  |  |
| Stop |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Closed User Group (CUG) |  |  |  |  |  |  |
| Invite |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Answer |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Bye |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Personal Network Management (PNM) |  |  |  |  |  |  |
| Invite |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Answer |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
| Bye |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |
|  | Customized Ringing Signal (CRS) |  |  |  |  |  |  |
| Stop |  | - | Immediate | Immediate | Not Applicable | Not Applicable |  |

Editor's note:	The use of IEC, PEC, ECUR and SCUR is FFS.

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event, Initial, Update, Termination] issued by the MMTel for these chargeable events, is specified in clause 5.4.3.

### 5.4.2	Message flows

#### 5.4.2.1	Introduction

The flows described in the present document specify the converged charging service with IMS NF (e.g. IMS AS) as the NF Consumer for different charging scenarios. The messages associated with these charging scenarios are shown primarily for information and to illustrate the charging triggers. 
They are not intended to be exhaustive of all the IMS message flows described in TS 24.228 [221].

Although each MMTel supplementary service is described by separated flows illustrating the dedicated trigger(s) for each MMTel supplementary service, the service may be combined with other MMTel supplementary services or IMS information.

The circumstances on which PEC, ECUR or SCUR are applied, depend on the MMTel supplementary service and/or operator's policy.

#### 5.4.2.2	Void

#### 5.4.2.3	Void

#### 5.4.2.4	Void

#### 5.4.2.5	Void

#### 5.4.2.6	MMTEL supplementary services – PEC

The message flows clause 5.2.2.1. except for figures 5.2.2.1.7.2.1, 5.2.2.1.9.1.1, and 5.2.2.1.9.2.1, are applicable with the following changes:

-	CDF replaced with CHF.

-	AS CDR replaced with CHF CDR.

-	Charging Data Request [Start] replaced with Charging Data Request [Initial].

-	Charging Data Request [Interim] replaced with Charging Data Request [Update].

-	Charging Data Request [Stop] replaced with Charging Data Request [Termination].

#### 5.4.2.7	MMTEL supplementary services – ECUR

The message flow in figure 5.3.2.1.4.1.1 is applicable with the following changes:

-	OCS replaced with CHF.

-	Addition of creation, updating, and closing of CHF CDR in CHF (OCS).

-	Reserve Units Request [Initial] replaced with Charging Data Request [Initial].

-	Reserve Units Request [Update] replaced with Charging Data Request [Update].

-	Reserve Units Request [Termination] replaced with Charging Data Request [Termination].

#### 5.4.2.8	MMTEL supplementary services – SCUR

The message flows clause 5.2.2.1. except the following figures 5.2.2.1.6.1.1, 5.2.2.1.6.2.1, 5.2.2.1.8.1, 5.2.2.1.10.1, 5.2.2.1.12.1, and 5.2.2.1.13.1, are applicable with the following changes:

-	CDF replaced with CHF

-	AS CDR replaced with CHF CDR

-	Charging Data Request [Start] replaced with Charging Data Request [Initial]

-	Charging Data Request [Interim] replaced with Charging Data Request [Update]

-	Charging Data Request [Stop] replaced with Charging Data Request [Termination]

The message flows clause 5.3.2.1, except the figure 5.3.2.1.4.1.1, are applicable with the following changes:

-	OCS replaced with CHF

-	Addition of creation, updating, and closing of CHF CDR in CHF (OCS)

-	Reserve Units Request [Initial] replaced with Charging Data Request [Initial].

-	Reserve Units Request [Update] replaced with Charging Data Request [Update]

-	Reserve Units Request [Termination] replaced with Charging Data Request [Termination]

### 5.4.3	CDR generation

#### 5.4.3.1	Introduction

The CHF CDRs for MMTel charging are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The following clauses describe in details the conditions for generating, opening, updating and closing the CHF CDR, which shall be supported by the CHF.

#### 5.4.3.2	Triggers for CHF CDR

##### 5.4.3.2.1	General

A MMTel charging CHF CDR is used to collect charging information related to MMTel chargeable events. The details of the triggers are specified in TS 32.260 [20].

### 5.4.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [54].

### 5.4.5	Bi CDR file transfer

Details of the Bi protocol application are specified in TS 32.297 [52].

# 6	Definition of charging information

## 6.1	Data description for MMTel offline charging

### 6.1.1	Rf message contents

#### 6.1.1.0	Introduction

For offline charging, the different  service CTFs supporting MMtel specific service charging generate accounting information transferred to the CDF using the Diameter accounting application, as described in TS 32.299 [50].

The Charging Data Request and Charging Data Response are used for this MMTel service offline charging, as described in TS 32.260 [20].

This clause describes the different fields used in the Charging Data messages and the category in the tables are used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

#### 6.1.1.1	Charging Data Request message description

The generic Charging Data Request message described in TS 32.260 [20] used for MMTel offline charging is enhanced with specific MMTel service information.

Table 6.1.1.1.1 illustrates the overall Charging Data Request message as used for MMTel offline charging.

# Table 6.1.1.1.1: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| See Charging Data-Request message fields described in TS 32.260 [20] |  |  |
| Service Information | OM | This field holds the MMTel specific parameter and is described in clause 6.3. |

NOTE:	Detailed descriptions of the fields are provided in TS 32.299 [50].

#### 6.1.1.2	Charging Data Response message description

The generic Charging Data Response message described in TS 32.260 [20] is used for MMTel offline charging.

### 6.1.2	GTP' message contents

Not applicable. Refer to sclause 5.2.3 for further information.

### 6.1.3	CDR description on the Bi interface

#### 6.1.3.1	CDR field type

The MMTel CDR content and format description is based on TS 32.260 [20] AS-CDR description, enhanced with specific MMTel service information.

The content of MMTel CDR type is defined in the table in clause 6.1.3.3. The field definition includes the field name, category and description. The category in the tables are used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1]. The detailed field descriptions are provided in TS 32.298 [51].

The CDF provides the CDRs at the Bi interface in the format and encoding described in TS 32.298 [51]. Additional CDR formats and contents may be available at the interface to the billing system to meet the requirements of the billing system, these are outside of the scope of standardization.

#### 6.1.3.2	CDR triggers

Accounting information for MMTel services charging is transferred from the CTFs supporting MMtel service charging to the CDF, using Charging Data Request start, interim stop or Event messages, depending on MMTel service behaviour (detailed in clause 5.2.2). Within the CDF, the CDRs triggering follows the same principle as described for SIP sessions in TS 32.260 [20]: MMTel CDR is opened upon reception of a Charging Data Request [start] message, closed upon reception of a Charging Data Request [stop] message, updated upon reception of a Charging Data Request [Interim], and created  upon reception of a Charging Data Request [event].

#### 6.1.3.3	MMTel-AS CDR content

The MMTel CDR content and format description is based on TS 32.260 [20] AS-CDR description, enhanced with specific MMTel information.

The detailed description of the field is provided in TS 32.298 [51].

# Table 6.1.3.3.1 : Charging data of MMTel CDR

| Field | Category | Description |
| --- | --- | --- |
| See Charging Data of AS CDR described in TS 32.260 [20]. |  |  |
| MMTel information | OC | This field includes a list of MMTel supplementary services which may occur within the same AS. The MMTel supplementary services related informations are described in clause 6.3.1.2. |

## 6.2	Data description for MMTel online charging

### 6.2.1	Ro message contents

#### 6.2.1.0	Introduction

For online charging, the different service CTFs supporting MMTel specific service charging, generate debit and reserve units information transferred to the OCF. For this purpose, MMTel online charging utilises the Debit Units and Reserve Units procedure specified in the Debit / Reserve Units operation in TS 32.299 [50].

The Debit / Reserve Units Request and Debit / Reserve Units Response messages are used for this MMTel service online charging, as described in TS 32.260 [20] and the category in the tables are used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

#### 6.2.1.1	Debit / Reserve Units Request message

The generic Debit / Reserve Units Request message described in TS 32.260 [20], used for MMTel online charging is enhanced with specific MMTel service information.

Table 6.2.1.1.1 illustrates the overall Debit / Reserve Units Request message as used for MMTel online charging.

# Table 6.2.1.1.1: Debit / Reserve Units Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| See Debit / Reserve Units Request message fields described in TS 32.260 [20]. |  |  |
| Service Information | OM | This field holds the MMTel specific parameter and is described in clause 6.3. |

NOTE:	Detailed descriptions of the fields are provided in TS 32.299 [50].

#### 6.2.1.2	Debit / Reserve Units Response message

The generic Debit / Reserve Units Response message described in TS 32.260 [20] is used for MMTel online charging.

## 6.3	MMTel charging specific parameters

### 6.3.1	Definition of MMTel charging information

#### 6.3.1.0	Introduction

The MMTel Information parameter used for MMTel charging is provided in the Service Information parameter.

#### 6.3.1.1	MMTel charging information assignment for Service Information

The components in the Service Information that are use for MMTel charging can be found in table 6.3.1.1.1.

# Table 6.3.1.1.1: Service Information used for MMTel charging

| Information Element | Category | Description |
| --- | --- | --- |
| Service Information | OM | A set of fields hold the 3GPP specific parameter as defined in TS 32.299 [50]. |
| Subscription Id | OM | Used as defined in TS 32.260 [20]. |
| IMS Information | OC | The details are defined in TS 32.260 [20]. |
| PS Information | OC | The details are defined in TS 32.260 [20]. |
| MMTel Information | OM | This field holds a set of MMTel services with theirs specific parameters. The details are defined in clause 6.3.1.2. |

#### 6.3.1.2	Definition of the MMTel Information

MMTel specific charging information is provided within the MMTel Information, and the detailed structure of the MMTel Information can be found in table 6.3.1.2.1

# Table 6.3.1.2.1 : Structure of the MMTel Information

| Information Element |  | Category |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Supplementary Service |  | OM |  | This is a grouped field comprising several sub-fields associated with one supplementary service. It can be present multiple times as necessary to present the parallel activity of the different supplementary services. |  |
| Service Type |  | OM |  | This field holds the type of the Supplementary Service, i.e. OIP, OIR, TIP, TIR, CW, HOLD, CB, MWI, CONF, CDIV, ECT, FA, MCID, CAT, CUG, PNM, CRS, AoC. |  |
| Service Mode |  | OC |  | This field holds the mode of specific Service Type, i.e. for CB: ACR, ICB, OC, Bfor CDIV: CFU, CFB, CFNR, CFNRc, CFNL, CFUDB and for CONF: 3PTY, AoC-S, AoC-D, AoC-E. |  |
| Number of diversions |  | OC |  | This field holds the number of diversions for CDIV. |  |
| Associated party address |  | OC |  | This field holds additional party identification needed for the service charging, i.e. for CDIV the "forwarding party", for ECT the "transferor", for FA the "Pilot Identity", for 3PTY the "Initiator party". |  |
| Service id |  | OC |  | This field holds the conference ID for CONF supplementary service |  |
| Change Time |  | OC |  | This field holds the time of the requested action indicated in the "Participant Action Type" during the CONF supplementary service. It provides the time stamps for the CONF supplementary service parameters reporting. When the action is set to "CREATE", this field indicates the start time of the CONF supplementary service. When the action is set to "QUIT" and Number Of Participants holds the value "0", this field indicates the end time of the CONF supplementary service. |  |
| Number Of Participants |  | OC |  | This field holds the number of parties who are currently attached to the Conference at the time stamped indicated in the "Change Time", for the CONF supplementary service. |  |
| Participant Action Type |  | OC |  | This field holds the participant action type for CONF supplementary service (CREATE_CONF, JOIN_CONF, INVITE_CONF, QUIT_CONF) at the time stamped indicated in the "Change Time". |  |
| CUG Information |  | OC |  | This field holds the CUG information conveyed by the Network and identifies the CUG-communication : it is the "CUG Interlock Code" . |  |
| AoC information |  | OC |  | This field holds the AoC related Charging information conveyed to CDF, as defined in TS 32.280 [69 Since AoC information may be sent more than once (e.g for AoC-S), this field may be multiple. |  |

#### 6.3.1.3	Support of MMTel Information in MMTel offline charging

In table 6.3.1.3.1, the supported Operation Types for Service Type field within the MMTel Information on the Rf interface are presented. The other MMTel Information fields are not detailed.

The supported Operation Types for Service Information fields, used in the Charging Data Request and Charging Data Response messages for MMTel charging, other than MMTel Information fields, are described in TS 32.260 [20].

Operation Types for Service Type, encompasses the various situations, where the basic MMTel service Charging Data Request messages are sent from:

-	a separate entity (another AS or S-CSCF), than the AS implementing the supplementary service;

-	 the same AS.

Each service may be reported through a dedicated Charging Data Request Message or combined Charging Data Request Messages (regrouped supplementary services or with basic MMTel service).

# Table 6.3.1.3.1: Supported values for Service Type in MMTel Information

| Information Element |  |
| --- | --- |
| Node Type | AS |
| Supported Operation Types | S/I/S/E |
| Service Type |  |
| OIP (see note 1) | SISE |
| OIR | SISE |
| TIP | SISE |
| TIR (see note 2) | SISE |
| CW | SISE |
| HOLD (see note 3) | SISE |
| CB (see note 2) | ---E |
| MWI | ---E |
| CAT | SISE |
| CCBS | SISE |
| CCNR | SISE |
| CONF | SISE |
| CDIV | SISE |
| ECT | SISE |
| FA | SISE |
| MCID | SISE |
| CUG | SISE |
| PNM | SISE |
| CRS | SISE |
| AoC | SISE |
| NOTE 1: Only present at terminating side. NOTE 2: Terminating side service. NOTE 3: AS may be involved for announcement purposes. |  |

#### 6.3.1.4	Support of MMTel Information in MMTel online charging

In table 6.3.1.4.1 the basic structure of the supported fields within the MMTel Information in the Debit and Reserve Units Request for IMS online charging on the Ro interface are presented. The Operation types are listed in the following order: I (initial)/U (update)/T (terminate)/E (event). Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire filed is not allowed in a node the entire cell is marked as "-".

# Table 6.3.1.4.1: Supported values in Debit / Reserve Units Request message MMTel Information

| Information Element |  |
| --- | --- |
| Node Type | AS |
| Supported Operation Types | I/U/T/E |
| Service Type |  |
| OIP (see note) | - |
| OIR (see note) | - |
| TIP (see note) | - |
| TIR (see note) | - |
| CW (see note) | - |
| HOLD (see note) | - |
| CB (see note) | - |
| MWI (see note) | - |
| CAT (see note) | - |
| CCBS (see note) | - |
| CCNR (see note) | - |
| CONF | IUTE |
| CDIV | IUT- |
| ECT | IUT- |
| MCID (see note) | - |
| PNM (see note) | - |
| FA | IUT- |
| CUG | IUT- |
| CRS (see note) | - |
| AoC (see note) | - |
| NOTE: Only reported in offline charging. |  |

## 6.4	Data description for MMTel converged charging

### 6.4.1	Message contents

#### 6.4.1.1	General

The Charging Data Request and Charging Data Response are specified in TS 32.290 [57] and include charging information. The Charging Data Request can be of type [Event, Initial, Update, Termination].

Table 6.4.1.1.1 describes the use of these messages for converged charging.

# Table 6.4.1.1.1: Converged charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | AS | CHF |
| Charging Data Response | CHF | AS |

#### 6.4.1.2	Structure for the converged charging message formats

##### 6.4.1.2.1	Charging Data Request message

Table 6.4.1.2.1.1 illustrates the basic structure of a Charging Data Request message as used for MMTel converged charging.

# Table 6.4.1.2.1.1: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| See Charging Data-Request message fields described in TS 32.260 [20] with MMTel specific triggers described in clause 5.4.1.2. |  |  |
| MMTel charging information | OC | This field includes a list of MMTel supplementary services specific information described in clause 6.4.2.2 |

##### 6.4.1.2.2	Charging Data Response message

Table 6.4.1.2.2.1 illustrates the basic structure of a Charging Data Response message as used for MMTel converged charging.

# Table 6.4.1.2.2.1: Charging Data Response Message Contents

| Information Element | Category | Description |
| --- | --- | --- |
| See Charging Data-Response message fields described in TS 32.260 [20] with MMTel specific triggers described in clause 5.4.1.2. |  |  |

### 6.4.2	Definition of the MMTel converged charging information

#### 6.4.2.1	General

The Charging Information parameter used for MMTel converged charging is provided in the following clauses.

#### 6.4.2.2	Definition of MMTel charging information

MMTel specific charging information used for MMTel converged charging is provided within the MMTel charging Information.

Table 6.4.2.2.1: Structure of MMTel Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Supplementary Service | OM | This is a grouped field comprising several sub-fields associated with one supplementary service. It can be present multiple times as necessary to present the parallel activity of the different supplementary services. |
| Service Type | OM | This field holds the type of the supplementary service: OIP, OIR, TIP, TIR, CW, HOLD, CB, MWI, CONF, CDIV, ECT, FA, MCID, CAT, CUG, PNM, or CRS. |
| Service Mode | OC | This field holds the mode of specific service type for - CB: ACR, ICB, OC, or B, - CDIV: CFU, CFB, CFNR, CFNRc, CFNL, or CFUDB and - CONF: 3PTY |
| Number of diversions | OC | This field holds the number of diversions for CDIV. |
| Associated party address | OC | This field holds additional party identification needed for the service charging for - CDIV the "forwarding party" - ECT the "transferor" - FA the "Pilot Identity" - 3PTY the "Initiator party". |
| Conference Id | OC | This field holds the conference ID specific for CONV |
| Participant Action Type | OC | This field holds the participant action type for CONF: CREATE_CONF, JOIN_CONF, INVITE_CONF, QUIT_CONF at the time stamped indicated in the "Change Time". |
| Change Time | OC | This field holds the time of the requested action indicated in the "Participant Action Type" during the supplementary service CONF. For "Participant Action Type": - CREATE this field indicates the start time of the CONF - QUIT and Number Of Participants is 0, this field indicates the end time of the CONF |
| Number Of Participants | OC | This field holds the number of parties who are currently attached to the Conference at the time stamped indicated in the "Change Time", for the CONF supplementary service. |
| CUG Information | OC | This field holds the CUG information conveyed by the Network and identifies the CUG-communication: it is the "CUG Interlock Code". |

#### 6.4.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by IMS node for IMS converged charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-TE). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.4.2.3.1 defines the basic structure of the supported fields in the Charging Data Request message for MMTel converged charging.

Table 6.4.2.3.1: Supported fields in Charging Data Request message

| Information Element | Node Type | IMS Node |
| --- | --- | --- |
|  | Supported Operation Types | IUTE |
| Session Identifier |  | -UTE |
| Subscriber Identifier |  | IUTE |
| NF Consumer Identification |  | IUTE |
| Invocation Timestamp |  | IUTE |
| Invocation Sequence Number |  | IUTE |
| Retransmission Indicator |  | IUT- |
| Notify URI |  | IU-- |
| Supported Features |  | IU-E |
| Service Specification Information |  | IUTE |
| Triggers |  | -UT- |
| Multiple Unit Usage |  | IUT- |
| Rating Group |  | IUT- |
| Requested Unit |  | IU-- |
| Used Unit Container |  | -UT- |
| Triggers |  | -UT- |
| IMS Charging Information |  | IUTE |
| MMTel Charging Information |  | IUTE |
| Supplementary Service |  | IUTE |
| Service Type |  | IUTE |
| Service Mode |  | IUTE |
| Number of diversions |  | IUTE |
| Associated party address |  | IU-E |
| Conference Id |  | I--E |
| Participant Action Type |  | IUTE |
| Change Time |  | -UTE |
| Number Of Participants |  | IUTE |
| CUG Information |  | I--E |

Table 6.4.2.3.2 defines the basic structure of the supported fields in the Charging Data Response message for MMTel converged charging.

Table 6.4.2.3.2: Supported fields in Charging Data Response message

| Information Element | Node Type | IMS Node |
| --- | --- | --- |
|  | Supported Operation Types | IUTE |
| Session Identifier |  | IUTE |
| Invocation Timestamp |  | IUTE |
| Invocation Result |  | IUTE |
| Invocation Sequence Number |  | IUTE |
| Session Failover |  | I--- |
| Triggers |  | - |
| Multiple Unit information |  | I--E |
| Result Code |  | IU-E |
| Rating Group |  | IU-- |
| Granted Unit |  | IU-- |
| Validity Time |  | IU-- |

#### 6.4.2.4	Formal MMTel converged charging parameter description

##### 6.4.2.4.1	MMTel charging CHF CDR parameters

The detailed definitions, abstract syntax and encoding of the MMTel charging CHF CDR parameters are specified in TS 32.298 [51].

##### 6.4.2.4.2	MMTel charging resources attributes

The detailed definitions of resources attributes used for MMTel charging are specified in TS 32.291 [58].

Annex A (informative):
Bibliography

This Annex is a placeholder for documents which are not explicitly cited in this specification.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Sep 2008 | -- | -- | -- | -- | Presentation to SA Plenary |  | 0.2.0 | 1.0.0 |
| Mar 2009 | SA-43 | SP-090058 | -- | -- | Presentation to SA for approval |  | 2.0.0 | 8.0.0 |
| Jun 2009 | SA-44 | SP-090432 | 005 | -- | Correction of offline charging message flow of Communications Diversion (CDIV) service | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SA-44 | SP-090432 | 007 | -- | Correction of offline charging message flow of Explicit Communication Transfer (ECT) service | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SA-44 | SP-090432 | 012 | -- | Clarification on MMTel charging architecture | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SA-44 | SP-090294 | 1 | - | Add "Flexible Alerting" MMTel supplementary service Charging description |  |  |  |
| Jun 2009 | SA-44 | SP-090294 | 2 | - | Basic principles description for MMTel online charging | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SA-44 | SP-090294 | 3 | - | R9 CR Add CCNR in MMTel Charging | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SA-44 | SP-090294 | 4 | - | R9 CR Add CCNR in MMTel Charging | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SA-44 | SP-090294 | 6 | - | Add MMTel CDIV online charging scenario | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SA-44 | SP-090294 | 8 | - | Add CCBS and CCNR MMTel supplementary services in Scope chapter. Small cleanups | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SA-44 | SP-090294 | 9 | - | Add "Malicious Communication Identification (MCID)" MMTel supplementary service Charging description | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SA-44 | SP-090294 | 10 | - | Add "Customized Alerting Tone (CAT)" MMTel supplementary service Charging description | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SA-44 | SP-090294 | 11 | - | Add "Closed User Group (CUG)" MMTel supplementary service Charging description | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SA-44 | SP-090294 | 13 | - | Add "Flexible Alerting" MMTel supplementary service Online Charging scenario | B | 8.1.0 | 9.0.0 |
| Jun 2009 | SA-44 | SP-090294 | 14 | - | Add "Closed User Group (CUG)" MMTel supplementary service Online Charging scenario | B | 8.1.0 | 9.0.0 |
| Sep-2009 | SA-45 | SP-090538 | 16 | - | Aligning MMTel Charging error cases with TS 32.260 IMS Charging | A | 9.0.0 | 9.1.0 |
| Sep-2009 | SA-45 | SP-090538 | 17 | - | Refinement on CUG information in MMTel information | F | 9.0.0 | 9.1.0 |
| Sep-2009 | SA-45 | SP-090538 | 19 | - | Setting for Operation Types related to "service type"values in ACR for MMTel charging | A | 9.0.0 | 9.1.0 |
| Sep-2009 | SA-45 | SP-090538 | 20 | - | Add 3PTY MMTel supplementary service charging | B | 9.0.0 | 9.1.0 |
| Sep-2009 | SA-45 | SP-090538 | 21 | - | Clarification on generic behaviour for Supplementary services Online Charging description | C | 9.0.0 | 9.1.0 |
| Dec 2009 | SA-46 | SP-090722 | 22 | - | Add "Conference (CONF)" MMTel supplementary service Online Charging scenario | B | 9.1.0 | 9.2.0 |
| Dec 2009 | SA-46 | SP-090722 | 23 | - | MMTel Online charging scenarios simplification – IMS-GWF interaction removal | C | 9.1.0 | 9.2.0 |
| Dec 2009 | SA-46 | SP-090722 | 24 | - | Add "CONF (3PTY)" MMTel supplementary service Online Charging scenario | B | 9.1.0 | 9.2.0 |
| Dec 2009 | SA-46 | SP-090722 | 25 | - | Add Messages description for MMTel Online Charging | B | 9.1.0 | 9.2.0 |
| Mar 2010 | SA-47 | SP-100044 | 26 | - | Add "Explicit Communication Transfer (ECT)" MMTel supplementary service Online Charging scenario | B | 9.2.0 | 9.3.0 |
| Mar 2010 | SA-47 | SP-100040 | 28 | - | Correction on MMTel CDR description for Ealy SDP - Alignment with TS 32.260 | A | 9.2.0 | 9.3.0 |
| Mar 2010 | SA-47 | SP-100040 | 30 | - | Correction in MMTel Charging for session priority - Alignment with TS 32.260 | A | 9.2.0 | 9.3.0 |
| Mar 2010 | SA-47 | SP-100044 | 31 | - | Add "Personal Network management" MMTel supplementary service charging description. | B | 9.2.0 | 9.3.0 |
| Mar 2010 | SA-47 | SP-100044 | 32 | - | Add "Customized Ringing Signal (CRS)" MMTel supplementary service charging description | B | 9.2.0 | 9.3.0 |
| Mar 2010 | SA-47 | SP-100044 | 33 | - | Add "Reverse Charging" MMTel supplementary service Charging not supported | B | 9.2.0 | 9.3.0 |
| Mar 2010 | SA-47 | SP-100044 | 34 | - | Completion for Supported values setting for MMTel Information - Online Charging | B | 9.2.0 | 9.3.0 |
| Sep 2010 | SA-49 | SP-100496 | 036 | -- | Correction to Message Sequence Chart for Offline Charging of CW Service | A | 9.3.0 | 9.4.0 |
| Sep 2010 | SA-49 | SP-100497 | 037 | -- | Correction to Support of MMTel Information in MMTel Online Charging | F | 9.3.0 | 9.4.0 |
| Dec 2010 | SA-50 | SP-100758 | 038 | 2 | Correction of Communication session Barring (CB) to support MMTel Information in MMTel Offline Charging | F | 9.4.0 | 9.5.0 |
| Dec 2010 | SA-50 | SP-100758 | 039 | 2 | Correction of Communication Waiting (CW) Charging Flow | F | 9.4.0 | 9.5.0 |
| Mar 2011 | SA-51 | SP-110109 | 42 | 1 | MMTel Charging enhancement for alignment with generic AS Charging description in TS 32.260 | C | 9.5.0 | 10.0.0 |
| Mar 2011 | SA-51 | SP-110112 | 40 | 1 | Add 'Advice Of Charge (AoC)' MMTel supplementary service Charging description - Align with 22.173 | B | 9.5.0 | 10.0.0 |
| Mar 2011 | SA-51 | SP-110109 | 41 | 1 | Correction on Subscriber role | F | 9.5.0 | 10.0.0 |
| 2012-09 | - | - | - | - | Update to Rel-11 version (MCC) | - | 10.0.0 | 11.0.0 |
| 2012-12 | SA-58 | SP-120793 | 42 | 1 | Single charging session applicability to MMTel | B | 11.0.0 | 11.1.0 |
| 2013-03 | SA-59 |  |  |  | Changes from SA-59 implemented |  | 11.1.0 | 11.2.0 |
| 2013-04 |  |  |  |  | History table correction by MCC |  | 11.2.0 | 11.2.1 |
| 2013-09 | SA-61 | SP-130464 | 44 | 1 | Correct use of PS information elements | F | 11.2.1 | 11.3.0 |
|  |  | SP-130437 | 47 | - | Correction on MMTel Online Charging | A | 11.2.1 | 11.3.0 |
| 2014-06 | SA-64 | SP-140336 | 52 | 1 | assured transfer charging discription | B | 11.3.0 | 12.0.0 |
| 2014-07 | - | - | - | - | Rapporteur/MCC: General editorial changes and clean-up. | - | 12.0.0 | 12.0.1 |
| 2014-09 | SA-65 | SP-140564 | 57 | 1 | Corrections for alignment between charging specifications | F | 12.0.1 | 12.1.0 |
|  |  | SP-140561 | 58 | 1 | Removal of CDIVN service | A |  |  |
| 2014-12 | SA-66 | SP-140804 | 59 | - | Additional corrections for removal of I-WLAN solution | F | 12.1.0 | 12.2.0 |
|  |  | SP-140805 | 60 | - | Corrections on definition for parameter category | F |  |  |
| 2015-03 | SA-67 | SP-150065 | 63 | 1 | Inconsistency correction of subscriber role | A | 12.2.0 | 12.3.0 |
| 2015-12 | SA-70 | SP-150816 | 65 | 2 | Introduction of Announcement service for MMTel service | B | 12.3.0 | 13.0.0 |
| 2016-03 | SA-71 | SP-160039 | 66 | 1 | Correction of interactions of Announcement service and MMTel supplementary services | F | 13.0.0 | 13.1.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-03 | SA#75 | SP-170138 | 0067 | 1 | C | Remove reference to RFC 3588 | 14.0.0 |
| 2018-06 | - | - | - | - | - | Update to Rel-15 version (MCC) | 15.0.0 |
| 2019-09 | SA#85 | SP-190760 | 0068 | 2 | B | Addition of converged charging architecture | 16.0.0 |
| 2019-09 | SA#85 | SP-190760 | 0069 | 2 | B | Addition of converged charging scenarios | 16.0.0 |
| 2019-09 | SA#85 | SP-190760 | 0070 | 2 | B | Addition of applicable Triggers for converged charging | 16.0.0 |
| 2019-09 | SA#85 | SP-190760 | 0072 | 2 | B | Addition of converged charging CDR handling | 16.0.0 |
| 2019-12 | SA#86 | SP-191164 | 0073 | 1 | B | Addition of converged flows | 16.1.0 |
| 2020-12 | SA#90e | SP-201068 | 0075 | 1 | B | Add service based architecture for offline charging | 17.0.0 |
| 2020-12 | SA#90e | SP-201068 | 0076 | 1 | B | Adding converged charging in relevance table | 17.0.0 |
| 2020-12 | SA#90e | SP-201068 | 0077 | 1 | B | Adding converged charging for supplementary service | 17.0.0 |
| 2021-03 | SA#91e | SP-210164 | 0078 | - | B | Introduction of CHF framework in the scope | 17.1.0 |
| 2021-03 | SA#91e | SP-210164 | 0079 | 1 | B | Introduce Post Event Charging (PEC) | 17.1.0 |
| 2021-06 | SA#92e | SP-210399 | 0080 | 1 | B | Adding CHF announcement handling | 17.2.0 |
| 2021-06 | SA#92e | SP-210399 | 0081 | 1 | B | Adding converged charging flows | 17.2.0 |
| 2021-06 | SA#92e | SP-210399 | 0082 | 1 | B | Adding converged charging data description | 17.2.0 |
| 2021-12 | SA#94e | SP-211482 | 0083 | 1 | F | Correction of references for converged charging | 17.3.0 |
| 2024-04 | - | - | - | - | - | Update to Rel-18 version (MCC) | 18.0.0 |